Installing GAMEDD.SYS from GAMEDD2.ZIP:

---------------------------------------

1)Install IBM joystick GAMEDD.SYS files as per instructions that come with

it (see JOYSTICK.ZIP).

2)Find the GAMEDD.SYS file and replace it with this one (see GAMEDD2.ZIP).

(you may want to keep a copy of original GAMEDD.SYS elsewhere for now)

3)In your CONFIG.SYS file you can add options to GAMEDD.SYS, example:

Normal default line found in config.sys (directory may be different)

DEVICE=C:\GAMEDD.SYS

These are the default values. Analog joysticks are default.

DEVICE=C:\GAMEDD.SYS /P:201 /W:400 /C:1E

This example allows two digital joysticks to work as if they are analog.

Gravis_Xterminator=joystick1, Microsoft_Sidewinder_Gamepad=joystick2,

and it was determined that the wait loop should be increased to 450 hex.

DEVICE=C:\GAMEDD.SYS /D1gx /D2mg /W:450

This example allows one digital joystick and one analog joystick.

Gravis_Xterminator=joystick1, Analog=joystick2, wait loop is default.

DEVICE=C:\GAMEDD.SYS /D1gx

Current options you can add in CONFIG.SYS:

------------------------------------------

/P:hhhh   (default is /P:201)

If you have an odd joystick port which uses port 301h or other location

you can use /P:hhhh where hhhh is the joystick I/O address (in hex).

/W:hhhh   (default is /W:400)

Default built-in WAIT is 400hex wait loops.

With today's very fast computers, you might not find the joysticks because

your computer stops looking before the joystick finishes doing it's work.

Some joystick ports run as much as ten times faster or more in some cases.

Some computers may be very fast yet have a very slow joystick port. For

example, you can only see an ISA slot AWE32 soundcard up to 66000 times a

second maximum because that is how fast the card can run regardless of if

it is a 386 or a PentiumXYZ. On the other hand, if you have a PCI soundcard

installed, you will probably need to use this /W option to increase the wait

counter. This appears to be mostly a problem with analog joysticks where for

example one user reported having to use /W:1600 for their analog joysticks.

The value you require depends on what sound card and joystick you have.

If you have a fast joystick port, then you may find this tip helpful.

You may want to slowly increase this value in the config.sys file until

you finally get propper joystick values. You will need to reset the computer

for the changes in the config.sys file to take effect. Suggestion is to

increase the /W: value in small increments like /W:600 /W:800 /W:A00 ....

You could put in something very large like /W:FFFF but it would put a huge

speed penalty on your computer and you may find your computer behaving

oddly since your computer will be ignoring all other processes while

waiting for a response from your joysticks. Joysticks are still a polled

device, so in other words /W:FFFF may be a bad idea.

For you speed demons out there... you may realize a pushbutton gamepad

will only go up to 200h or some small number....therefore you may want to

use a /W:250h to decrease the waiting time for non-existant joysticks.

Why waste time? Suggestion is about 20% more than maximum value your

joystick reports back.

Digital joysticks require time to fill the built in buffer, because they

are also a polled device. Follow the same steps above if you are getting

intermittent joystick readings.

Note: some digital joysticks run faster than the joystick port can handle,

therefore you may need to replace your joystick port in those cases.

For example, the Microsoft Sidewinder runs faster than a Creative Labs

AWE32-ISA-Slot soundcard joystick port, therefore you will get

intermittent output regardless of the /W option.  Choose another faster

joystick port or choose a different digital joystick.

/C:hh	(default is 30%, which in hex values is 1E)

You should center your joysticks before calling a program that uses the

joysticks. "Calibration" happens when the computer powers up, therefore

your joysticks should be centered before you power up your computer.

Calibration is a useful way of making an analog style joystick act like

a pushbutton gamepad. For the default of 30%, here is what you get using

the X-axis of 100,0=center as an example:

lower trip point becomes 100-30%=70, upper trip point becomes 100+30%=130.

0,0-----------100,0-----------200,0

|	|     |

70,0  100,0 130,0

<--lower->|<--center->|<-right->

50	  |    100    |     150

When your joystick X-axis is within 70...130, the joystick returns 100,

and when your joystick X-axis is below 70, it returns 50, while above 130,

it will return 150.

To make your joystick appear more sensitive, try 10%=0A or 5%=5, and you

will notice that you do not need to move the joystick much to give a

digital response. A larger number means you need to move your stick more

before returning a lower/upper value. You are allowed to set the

calibration anywhere from 10% to 45% ( /C:A .... /C:2D in hex) otherwise

it is set to default of 30%.

Here is graphical examples of 45% and 10% calibration values for X-axis:

0,0-----------100,0-----------200,0

|	|	|

55,0    100,0   145,0

<--lower|<----center--->|right->

50	|      100	|   150

0,0-----------100,0-----------200,0

|	|  |

90,0 | 110,0

<-----lower->|<-c->|<-right---->

50   | 100 |   150

You can see this happen for JOYS_DOS.EXE by changing the "DOS SETTING" of

"GAME_DIGITAL_RESPONSE" to ON and "GAME_DIRECT_ACCESS" to OFF, then run

JOYS_DOS.EXE.  Note, you must have GAMEVDD.SYS installed to simulate the

joystick port for DOS programs (see JOYSTICK.ZIP by IBM). Users with PCI

cards will have to have GAMEVDD.SYS installed if their joystick port is

set at a value that is not 201h.

/D1xx and /D2xx

Digital joystick1 or digital joystick2, you may mix and match certain

sticks. Please set your DOS games to read the simulated values from

gameport 201h and not from BIOS since BIOS appears to give wrong results

for digital sticks. For DOS games and the older OS2 gameport driver IO,

this is what you get:

/D1xx options return 4 pushbuttons if no second joystick is attached.

/D1xx options return 2 pushbuttons if a second joystick is attached.

/D2xx options return 2 pushbuttons only.

Definitions for different joysticks:

/D1gp or /D2gp  Gravis Gamepad Pro		verified

/D1gb or /D2gb  Gravis Blackhawk		need volunteer to test

/D1gx or /D2gx  Gravis Xterminator		verified

/D1mg or /D2mg  Microsoft Sidewinder Gamepad	verified by Bret Johnson

Note: Current sidewinder algorithm re-writes own buffer space, therefore

do not install a second digital joystick as /D2xx. If you own 2 digital

joysticks then install sidewinder as /D2mg, for example /D1gx /D2mg works

fine. /D1mg /D2?? won't work. This routine will be re-written in future

so that this condition is fixed, but for now, enjoy.

PCI Soundcards And The Joystick Port Help Tips:

-----------------------------------------------

Some users have reported trouble in installing the joystick driver on the

newer PCI style sound cards versus the older ISA style sound cards. There

are a few things that you need to consider. With the older ISA style

sound cards, the joystick port tended to be located at 201h, therefore you

did not have to use the /P:xxxx option since 201h was the default location.

With some newer PCI sound cards, the joystick may be located at a much

higher address and you will need to use the /P:xxxx option to set it, for

example the address may be /P:DC00 or /P:EF00 or some other address.

Currently, I haven't written any programs to help you find this address,

but if you look for the latest SoundBlaster drivers, I believe there is a

utility included with it to help you find the location of the PCI joystick

port. If you have a PCI soundcard with a high joystick address, you will

need to get IBM's GAMEVDD.SYS if you do not already have it (located on

JOYSTICK.ZIP) and install it on your computer to translate the GAMEDD.SYS

data to a virtual 201h address for DOS programs since DOS programs usually

expect to see 201h as the normal joystick address.

Some users have mentioned their PCI sound cards appear not to work at the

new PCI address. It might work fine for users who have analog gamepads,

but users with joysticks may have trouble since analog joysticks may take

longer to collect a valid reading. This is difficult to verify with a digital

joystick at this time because nothing appears to work. It is much better

if you ask a friend to borrow their analog joystick first then for you to

run GAMEDD.SYS as an analog joystick driver first. If you have pushbutton

access with analog joysticks, then you have the correct /P:xxxx joystick

port address. If the X/Y axis do not function correctly, then you will need

to fine tune the /W option. One user mentioned having to change the /W

option to /W:1600 to get their joystick to work (use that as a suggestion

since your computer and joystick combination may need a different /W value

to work). Once the /P and /W options seem to work correctly, leave them as

is if you decide to later install digital joysticks. The /W option may be

something you want to consider because even though computers and PCI sound

cards run much faster, joysticks still require time to process. Follow the

/W option tips above to tune your port to minimize CPU wasted time.

Digital Style Joysticks And DOS Games:

--------------------------------------

DOS games are looking for generic style linear Joysticks and will not know

what to do with the information even if your joystick port is at the default

location of 201h. You will require IBM's GAMEVDD.SYS located on JOYSTICK.ZIP

to translate GAMEDD.SYS information to a linear style which can be used by

DOS games.

GAME$ Programming:

------------------

For those of you wishing to write programs to use the joystick port, you

may want to look at JOY_IBM.CPP for programming help. This example should

be compatible to IBM's JOYSTICK.ZIP driver of 1995, so it should hopefully

help you be compatible from 1995 up to today in terms of GAME$ access.

GAME$ Compatibility Chart:

--------------------------

As of Feb 10, 2002, GAMEDD2.ZIP is backwards compatible with GAMEPORT.SYS,

GAMEPRT2.SYS, and IBM's GAMEDD.SYS device drivers. It is now also partially

compatible with Vesa's GPP_DRIV.ZIP.

1989,1992	1999,2000

GAMEPT.ZIP	GAMEPRT2.ZIP

GAMEPORT.SYS--->GAMEPRT2.SYS--------+

Frank Ivan	Jose DaSilva	    |		^ 80286 or better

V		-----------------

19??..1995			1999..2002	v 80386 or better

JOYSTICK.ZIP,JOYDOCS.ZIP	GAMEDD2.ZIP

GAMEVDD.SYS-----+-------------->(use original GAMEVDD.SYS)

GAMEDD.SYS------|+------------->GAMEDD.SYS

IBM, ver2.0	||		Jose Da Silva, Canada

||

||  1999

||  GPP_DRIV.ZIP,EJOYOS2_V21.ZIP

|+->GAMEDD.SYS

+-->GAMEVDD.SYS

Vesa Jskelinen, Finland

History:

--------

Feb 10,2002 Now compatible with extended instructions 21h,23h..28h

Nov 19,2001 Added JOY_IBM.CPP joystick.zip compatible programming example.

Jun 28,2001 Button config /D1gp & /D2gp. DOS Digital Response fixes.

Jun 12,2001 Tested/debugged /D1gp /D2gp /D2gx.

Jul 16,2000 Now backwards compatible to GAMEPORT.SYS & GAMEPRT2.SYS

May 05,2000 Added /D1 and /D2 options for more digital joysticks.

Mar 29,2000 Added /D1 option for Gravis Xterminator.

Jan 13,2000 Added /C and /S option for Analog Joysticks.

Dec 30,1999 Added /P and /W options.

Files in GAMEDD2.ZIP:

---------------------

GAMEDD2.DOC  = This file.

GAMEDD.SYS   = OS/2 Joystick Device driver (to replace IBM's GAMEDD.SYS).

JOYS_DOS.EXE = DOS test program (must have GAMEVDD.SYS installed).

JOY_PORT.EXE = OS/2 test program (GAMEPORT.SYS compatible).

JOY_PORT.CPP = Source code (GAMEPORT.SYS compatible, simple but nonstandard).

JOY_IBM.CPP  = IBM <joyos2.h> compatible program example for programmers.

Thanks To These People For Their Help:

--------------------------------------

Sep 09,2001 Ruediger Ihle and Sander van Leeuwen for info on PCI soundcards

and showing info on how to connect joystick drivers to SBlive.

May 05,2000 Bret Johnson for his sidewinder routines of /D1mg and /D2mg.

Mar 29,2000 Vojtech Pavlik of SuSE labs (Linux joystick drivers fame) for

helping in digital algorithms for Gravis digital joysticks.

These are options /D1gp /D1gx /D1gb, /D2gp /D2gx and /D2gb.

Dec 30,1999 Jason Stefanovich for hosting the contest and website where

GAMEPRT2.ZIP and GAMEDD2.ZIP were initially located.

Minimum tested system:

486 computer at 40MHz

by J DaSilva, Silva@JoesCat.com, Feb10/2002