SB16_IDE.SYS   driver version 1.0

=================================

In 1995 Creative Technology Ltd. redesigned their sound cards in order to

replace all jumper settings by Intel/Microsoft's "plug and play" software

mechanism. If your computer's ROM contains a PnP BIOS there should be no

problem to use SoundBlaster PnP cards with OS/2. If your computer has no

PnP BIOS you need special software drivers to activate your sound card after

power on. While you can download official OS/2 drivers for the sound portion

of SoundBlaster PnP cards, there exists currently no support for the

built-in IDE (CDRom) controller.

SB16_IDE.SYS is not an official OS/2 driver but can be considered only as a

"temporary fix" to enable the IDE CDRom controller on Creative's SoundBlaster

PnP cards. It does not implement full PnP functions nor does it check for

hardware conflicts. It only verifies the presence of the appropriate hardware

and initializes the card's PnP registers to predefined address settings.

After loading this driver the SoundBlaster card's IDE controller part can

be recognized by IBM's standard IBM1S506.ADD driver.

Installation:

1. Verify that the following I/O addresses and IRQs are free (RMVIEW.EXE):

I/O port 20Bh                       (for PnP configuration)

and I/O port 170h + port 376h + IRQ 15  (for IDE controller, option 2)

or          1E8h +      3EEh +     11  (       ...        ,  ...   3)

or          168h +      36Eh +     10  (       ...        ,  ...   4)

2. Copy SB16_IDE.SYS to \OS2 or \OS2\BOOT of the OS/2 boot partition.

3. Add one line to CONFIG.SYS:

BASEDEV=SB16_IDE.SYS 2          (exactly one space before number)

or BASEDEV=SB16_IDE.SYS 3

or BASEDEV=SB16_IDE.SYS 4

4. Verify or update the following CONFIG.SYS entries:

BASEDEV=IBM1S506.ADD /V                      (option 2 only)

or BASEDEV=IBM1S506.ADD /V /A:1/PORT:1E8/IRQ:11 (option 3 only)

or BASEDEV=IBM1S506.ADD /V /A:1/PORT:168/IRQ:10 (option 4 only)

and BASEDEV=IBMIDECD.FLT                (if IDE CDRom connected)

and DEVICE=<path>\OS2CDROM.DMD /Q       ( ...                  )

Exact syntax of options may be different depending on driver versions

and your hardware configuration.

5. Reboot

SB16_IDE.SYS is FREEWARE. It has been written without any support or permission

from Creative Labs and/or IBM, use it at your own risk. I could only test it

with WARP Connect and SoundBlaster16PnP. If you have problems or if you like to

receive the source code, send me an e-mail to 100021.313@compuserve.com .

Additional remarks:

- SB16_IDE.SYS has only been tested with SoundBlaster16PnP, but most likely it

works with other SoundBlaster PnP family models. It will definitely not work

with compatible sound cards from other manufacturers.

- SB16_IDE.SYS does not care about the type of CDRom drive connected to the

SoundBlaster card. You can only connect CDRom drives fully compatible to the

EIDE/ATAPI specification that are known to be supported by IBM1S506.ADD .

- Do not load SB16_IDE.SYS if a PnP BIOS is present.

- Check for I/O + IRQ conflicts, your computer may hang if there are conflicts.

- During a ctrl-alt-del warm reboot the SoundBlaster PnP IDE port remains active

even without this driver.

- Without a true PnP BIOS its not possible to boot OS/2 from a secondary hard

disk connected to the SoundBlaster PnP card, SB16_IDE.SYS can't help.

- If you want to install OS/2 from a CDRom connected to the SoundBlaster card,

you must include above CONFIG.SYS changes to the second boot floppy disk.

Include additionally the line "SET COPYFROMFLOPPY=1" to CONFIG.SYS and copy

SB16_IDE.SYS to the root directory of this floppy disk.

- It is possible to change the predefined address of the PnP configuration port

from 20Bh by patching SB16_IDE.SYS at offset 21Ah. (203h...3FFh step 4 allowed)

- For users of IBM's new hires timer driver TIMER0.SYS only: Because I don't

know what it does exactly to the hardware timer chip, it is a safe solution

to place BASEDEV=SB16_IDE.SYS before BASEDEV=TIMER0.SYS in CONFIG.SYS .

- "SoundBlaster","SoundBlaster16PnP", "PnP", "IBM", "OS/2" and "WARP Connect"

are trademarks or registered trademarks of their respective owners.

** Latest news: SB16_IDE.SYS has been reported OK with SoundBlaster AWE32PnP **

Robert Lalla, Loerrach, Germany       5-19-1996 (updated 6-23-1996)