Bugs Problems & things To Be Done & solutions.

==============================================

General stuff

=============

When setting up for display controller work from 640 * 480 upwards. As 640*480

is the easiest to get working.

If you have problems then it is most likely due to problems in setting up

your TV using the settings notebook. PLEASE READ THE ON-LINE HELP.

Our personal preference is to use an 800 by 600 display resolution with 64K

colours. 256 colours works fine but capture looks poor, 64K colours on the

other hand gives 'almost' photo-realistic captures.

If you are using any of the MMPM/2 utilities and you have selected > 256

colours then colour keying will not work correctly, the result will be that

you cannot see the TV (WarpTV does not suffer from this problem). This is

a bug & hopefully will be fixed.

If you get completely stuck, then officially you should contact Hauppauge

Computer Works Inc. Tech support : 001 516 434 3197

The latest releases of the drivers will always be placed on the Hauppauge BBS

first. 001 516 434 8454. The drivers may be available from other sources.

If you think you have spotted a bug, or need to get in touch with the authors

of the device drivers, then they can be reached at internet address :

abbot1@ibm.net

============================================================================

For Celeb driver Rev: 3.001B

Date: 1/9/95

This driver has been tested for all resolutions upto & including 1280 by 1024.

We can only guarantee operation on 640 by 480 by 256 colours, but hopefully the

driver should work with all display controllers at all resolutions.

In our experience ATI display controllers can cause problems, if you have a

problem it is most likely to be because the setup page is not calculating

the HSync and/or VSync polarity correctly (the symptom is a black TV window).

To fix it go to the VGA Sync settings page & experiment with the HSync & VSync

settings.

======================================================================

For Prism driver Rev: 2.001B

Date: 1/9/95

Contrast - does nothing, this will not change.

The TV image is currently restricted to 800 by 600 pixels, take it beyond that

& it will break up.

The TV image will only support colour resolutions upto & including 64K.

16 million colours is NOT supported on the Prism.

======================================================================

For HighQ driver Rev: 3.001B

Date: 1/9/95

This is the Celebrity driver with tuner functions removed. All comments re

Celebrity apply

=======================================================================

Good Luck Tim