****************************************************

*						       *

*	  200E PLUS Series NDIS Driver Installation    *

*						       *

****************************************************

Install NDIS 2.0.1 Driver for PC-NFS

========================================

1. Prepare the "QuickNDIS with drivers for PC-NFS" diskette and insert

it into floppy diskette drive A: or B:.

2. Run QUICKNDIS command.

3. After the "NDIS Driver" menu appears, choose "zero" for the third

party driver.

4. Next, input path and make sure the driver, filename with extension NIF,

and PROTOCOL.INI are in the diskette drive with path "NDIS_20".

5. Confirm whether the driver name is correct by "CNet CN200E Plus Series

Ethernet Adapter" message.

6. When the driver is installed, you are required to configure the

IRQ, I/O address, and driver name. Default values are 0x300, 3, and

E200P$.

7. QUICKNDIS program will update the CONFIG.SYS and AUTOEXEC.BAT

files automatically.  After these operations, reboot the workstation.

The network card driver will be loaded into memory.