****************************************************

*						       *

*	 200E PLUS Series NDIS Driver Installation     *

*						       *

****************************************************

PC/TCP for OS/2 v1.2 pl 1

===============================

Step 1 : Copy A:\NDIS_20\E200P.OS2 to C:\PCTCP\DRV

(where "A:" is the drive this diskette is in)

Step 2 : Add the following line to the CONFIG.SYS file :

DEVICE=C:\PCTCP\DRV\E200P.OS2

For example,

DEVICE=C:\PCTCP\DRV\PROTMAN.OS2 /I:C:\PCTCP\DRV

DEVICE=C:\PCTCP\DRV\E200P.OS2

DEVICE=C:\PCTCP\DRV\SOCKET.OS2

Step 3 : Edit the C:\PCTCP\DRV\PROTOCOL.INI file:

For example,

;************************

;**	 ADAPTERS	  **

;************************

;  200E PLUS Series of cards

;

[E200P_NIF]

DRIVERNAME = E200P$

INTERRUPT = 3

IOBASE = 0x300

;************************

;**	   PROTOCOLS	  **

;************************

;FTP PC/TCP for OS/2

[SOCKET]

drivername = $SOCKET

bindings = E200P_NIF

PC/TCP for DOS v2.2

=========================

Step 1 : Create the directory named LANMAN in hard disk which consists of

relevant files from the directory of NDIS of the diskette label

of "PC Network Software Unsupported Network Software Diskette A".

Step 2 : Run INSTALL.EXE in PCTCP directory.

Step 3 : Choose "NDIS Driver" item from the "Network Interface Card

Driver Selection" window. Then, press F10.

Step 4 : Input driver pathname "\NDIS_20" from driver diskette in the

"Interface Driver" window.

Step 5 : Key in complete pathname with file of "C:\LANMAN\PROTMAN.DOS".

Step 6 : Enter complete pathname with file of "C:\LANMAN\NETBIND.EXE".

Step 7 : Modify the content of protocol.ini listed below :

[E200P] ------------------------->  [E200P_NIF]

Step 8 : Append the content of CONFIG.INS file in PCTCP directory to

C:\CONFIG.SYS. Add one more option in the following statement.

DEVICE=C:\LANMAN\PROTMAN.DOS  ------------------> Old Version

DEVICE=C:\LANMAN\PROTMAN.DOS /I:C:\LANMAN	-----> New Version

Step 9 : Copy the PCTCP\AUTOEXEC.FTP to AUTOEXEC.BAT