USER'S GUIDE FOR THE PDS IMDISP PROGRAM

Chapter 1

INTRODUCTION

This document describes version 7.9g of the IMDISP program, an

interactive image processing utility for the IBM Personal Computer family

(PC, XT and AT) and compatibles.  It has been developed for use with the

Compact Disk - Read-Only Memory (CD-ROM) storage systems currently being

evaluated by the Planetary Data System (PDS).  It can also be used to

display and process images stored on floppy or hard disks.

This User's Guide presents an overview of image processing and CD-ROM

fundamentals, a section on the installation of IMDISP and CD-ROM software

and hardware, a description of IMDISP program operation, a description of

other utility programs which can be used in conjunction with IMDISP, and a

command summary.

Other support software for using the PDS CD-ROM disks has also been

developed.  These packages include the USGS Planetary Image Cartography

System (PICS) for MicroVAX computers, a set of FORTRAN routines to access

CD-ROM disks on VAX computers (VMSCD), and "C" language source code to

process disks written in the High Sierra format.  Contact Mike Martin

(mmartin@jplpds.jpl.nasa.gov) for access to these tools.

Chapter 2

BACKGROUND

Efforts to utilize small computer systems for display and analysis of

scientific data have been hampered by the lack of sufficient data storage

capacity to accommodate large image arrays.  Most planetary images require

nearly a megabyte of storage.  Over the past two years a new storage

technology, "CD-ROM", has been developed which provides the capability to

store up to 600 megabytes of data on a single 4.72-inch disk.  The disks

can be mass produced, and hundreds of copies of large digital archives

distributed rather than the few sets currently produced.  Readers for

CD-ROM disks can now be purchased for under a thousand dollars for use on

personal computers and some science workstations.

The PDS has worked with industry and other government agencies to

support the development of CD-ROM as a major data distribution and storage

media for space science data.  Initial development efforts by the PDS, the

Voyager Project, and multi-disciplinary science representatives have

resulted in the production of 12 CD-ROM disks containing about 30,000

images from the Voyager 1 and 2 encounters.  The IMDISP program can be used

to display most of these data files on a personal computer equipped with a

CD-ROM drive.

Any feedback on the use of IMDISP is greatly appreciated, and any of the

following people can be contacted for reporting bugs, adding enhancements,

etc:

Mike Martin

Jet Propulsion Lab

Internet: mmartin@jplpds.jpl.nasa.gov

SPAN: JPLPDS::MMARTIN

Ron Baalke

Jet Propulsion Lab

Internet: baalke@kelvin.jpl.nasa.gov

Archie Warnock

Goddard Space Flight Center

SPAN:     NDADSA::WARNOCK

NSSDC::WARNOCK

Internet: warnock@hypatia.gsfc.nasa.gov

warnock@nssdc.gsfc.nasa.gov

2.1 DIGITAL IMAGE PROCESSING SUMMARY

A digital image is a picture converted to numerical form so that it can

be stored and used in a computer.  The image is divided into a matrix of

small regions called picture elements, or pixels.  The rows and columns of

pixels are called "lines" and "samples", respectively.  Each pixel has a

numerical value, or DN (data number) value, quantifying the darkness or

brightness of the image at that spot.  In total, each pixel has an address

(line number, sample number) and a DN value, which is all that the computer

needs for processing.  The DN value of each pixel usually represents a

shade of darkness or brightness between black and white (gray levels).  How

many gray levels there are in an image depends on the number of bits used

to represent each pixel intensity in the computer.  The number of gray

levels will be equal to 2^n, where n is the number of bits per pixel's DN

value.  If 8 bits are used to represent a pixel's DN value (gray level),

the system will be capable of using 2^8, or 256, gray levels in an image,

where DN 0 is pure black, and DN 255 is pure white.  If each DN used only 4

bits of storage, the image would contain only 2^4 or 16 gray levels; if

there were only 1 bit per DN, the image would contain only black and white

pixels (bit values of 0 or 1).

2.2 OVERVIEW OF IMDISP CAPABILITIES

While the specifics of IMDISP commands are discussed in Chapter 4, this

section provides a general overview of how a user can manipulate images

once a file is selected.  There are DISPLAY commands which allow the user

to display all or part of an image at various positions on the display

screen.  The user may also zoom in and out from a point on the image

defined by the cursor, and may pan around the image.  The cursor may be

turned on or off; when on, the arrow keys are used to move the cursor

around the screen; when off, the cursor is not visible.

A user may choose to "subsample" the image, to enable more or all of the

original image to be displayed on the screen at once.  For example, if the

image were subsampled by a factor of 2, every other pixel from every other

line would be displayed, starting from the upper left corner of the image.

If a factor of 3 were chosen, every third pixel from every third line would

be displayed.  Any positive integer may be used for subsampling.

The user may produce a histogram of an image file, which is a graph

showing the number of pixels per DN value, or per range of DN values, for

the entire image.  Histograms are overlaid on top of the image, but may be

removed using the REFRESH command.  Subsampling may be specified when

computing a histogram to reduce the time required for the calculations.

A profile may be created which plots DN value versus pixel along a line

between two points on the image.  After the cursor is used to select the

two endpoints, the connecting line is drawn on the image and the profile is

plotted at the bottom of the screen.  There is a plot command to plot the

actual data values in an image line.

The user may also "stretch" the image, which is analogous to turning the

contrast knob on a TV set.  The user specifies low and high DN values; all

pixels with values lower than the specified "low" become black, and all

pixels higher than the specified "high" value become white.  All pixels

between the low and high values are evenly shaded between black and white.

IMDISP also has several filter functions available.  The user may process

the image with a convolution filter, mean or median filters.  There is also

an edge detection filter, as well as functions to brighten, darken and

smear the displayed image.

There are bookkeeping types of commands, also.  These allow the user to

do such things as retrieve a desired image from the CD-ROM, to save an

image to a file on hard disk, and to erase only the graphics from the

screen or to erase the entire screen.  Users may execute batch command

files to do a series of tasks automatically.  Lastly, the user may, of

course, EXIT the IMDISP program when done.

2.3 GRAY LEVEL AND FALSE COLOR IMAGES

Most planetary images are composed of 8-bit DN values representing

monochrome brightness levels in the scene.  To obtain color images,

separate images are taken through color filters (red, green, blue) and are

then combined by ground processing systems to produce a true color image.

Only a very small fraction of planetary images are available in color

versions.  Most display and analysis is done on monochrome images.

A display with 256 (2^8) gray levels is required to present the

information contained in a standard image.  However, the human eye can only

distinguish about 32 gray levels.  Thus 5-bit DN values would satisfy most

display requirements.  Unfortunately, computers are oriented to the storage

of and manipulation of items which are a power of two, and 5-bit pixels

would be very clumsy.  The standard display devices which the IMDISP

program supports are limited to 16 gray levels (PGA and VGA), 4 gray levels

(EGA) or 2 gray levels (CGA), but the program also supports several

"enhanced" VGA display devices offering up to 256 colors (or 64 shades of

gray).  The 16 gray levels of the PGA are adequate to support image

analysis; however 4 or 2 gray levels are practically useless for viewing

planetary images.  Fortunately the EGA display will support 16 different

colors, and a color palette can be selected which uses a graduated scale of

colors to represent gray levels, producing a "false color" image (false

because the displayed color does not represent the actual color of the

scene).

A false color image (also called a pseudo color image) is created from a

black and white image by assigning a color (rather than a gray level) to

each DN value in the image.  For instance, a DN value of 128 could be

reassigned to yellow if the user so desired.  Ranges of DNs (e.g., 100-125)

may also be assigned one color.  Pseudo colors get assigned to DN values in

a pseudo color table, which the display program then uses to determine how

to color the image on the monitor.  Display programs often have preset

pseudo color tables with commonly used DN-color combinations, and the user

can simply call for one of these when generating a pseudo color image.  The

option exists, of course, for users to generate their own pseudo color

tables.  Pseudo colors are often used to highlight features of an unusual

nature in an image.

Since the Enhanced Graphic Adapter for the IBM PC provides only 4 gray

shades (black, dark gray, light gray and white) most image viewing is done

using a pseudo color table which interprets gray levels as color values

ranging from black through reds, greens and blues up to white.

2.4 RADIOMETRIC AND GEOMETRIC CORRECTIONS

All camera systems have some kind of distortions in the lens, in the way

the shutter works, etc.  Distortions in the amount of light transferred

through the camera to the imaging plate are called radiometric distortions.

Geometric distortion affects the "squareness" of an image, and is detected

by taking an image of a geometric grid.

For spacecraft cameras, both types of distortion can be measured before

launch, and in a limited fashion, in flight.  These measured distortions

are saved in digital files.  When a raw image is received, these

"calibration files," as they are called, are subtracted from (or otherwise

applied to) the image, effectively removing radiometric and geometric

distortion.  This process is called radiometric and geometric correction.

Corrected images contain the closest possible representation of the scene

being imaged.

2.5 IMAGE FILE FORMATS

Digital image files are seldom stored as a simple array of pixels.

Generally there is an area at the beginning of an image file containing

descriptive information about the image.  This is referred to as a label or

header area.  Figure 2-1 shows a diagram of a simple image format with a

label area followed by the image lines.  In practice, images from planetary

missions have a more complicated format, due to the need to store

additional information to allow proper interpretation of the image data.

Figure 2-2 shows the format of a Voyager image, which has engineering

parameters embedded at the end of each image line to form a line suffix

area, an engineering trailer record after the last image line, and this is

followed by an image histogram.

������������������������Ŀ

� Labels or Header Area  �

������������������������Ĵ

� Image Array Line 1     �

������������������������Ĵ

� Image Array Line 2     �

��������������������������

.

.

.

������������������������Ŀ

� Image Array Line n     �

��������������������������

Figure 2-1: Simple Image Format

Different image label formats have been developed for use by nearly all

image processing facilities.  Two formats are widely used within the

planetary and astronomy communities: the VICAR2 (Video Image Communication

and Retrieval) labels used by the Multi-Mission Image Processing Laboratory

(MIPL) and FITS (Flexible Image Transport) labels used for astronomy image

interchange.  The PDS has developed a label scheme which is very similar to

these standards, called the Object Description Language (ODL).  This format

attempts to add a broader range of data descriptive capabilities to the

proven capabilities of the existing label systems.

Most of the images stored on the PDS CD-ROM disks have either ODL or

VICAR2 labels.  IMDISP will automatically interpret these labels to

determine the display format of the image data.

������������������������������������Ŀ

� PDS ODL Labels                     �

������������������������������������Ĵ

� Image Array Line 1  � Line Suffix  �

������������������������������������Ĵ

� Image Array Line 2  � Line Suffix  �

��������������������������������������

.

.

.

������������������������������������Ŀ

� Image Array Line 800 � Line Suffix �

������������������������������������Ĵ

� Image Engineering Trailer          �

������������������������������������Ĵ

� Image Histogram                    �

��������������������������������������

Figure 2-2: Voyager CD-ROM Image Format

2.6 CD-ROM FUNDAMENTALS

The CD-ROM disk uses the same basic data storage format as a CD audio

disk.  In fact, data blocks are identified by minute, second and sector

number, following the audio format.  Data blocks are recorded along a

spiral from the inner to the outer radius of the disk.  Each raw data block

(sector) contains 2,352 bytes of information, with 304 bytes used for

housekeeping and error correction and 2,048 (2K) containing user data.

Each data block is called a sector, and 75 sectors are stored per second.

Since the nominal playing time of a CD disk is 60 minutes, the data storage

capacity is 75 sectors per second * 60 seconds per minute * 60 minutes or

270,000 sectors.  Thus the nominal storage capacity of a single CD-ROM disk

is 540,000 kilobytes, which can be extended to more than 600,000 kilobytes

and beyond by recording more than 60 minutes of data on a disk.

In order to maximize the storage capacity of CD disks a constant linear

velocity (CLV) recording format is used.  This means that the player

changes speed (slows down) as it reads from the inside to the outside of

the disk, to maintain a constant flow of data under the read mechanism at a

speed of 1.2 meters per second.  This is in contrast to most magnetic disk

drives which use constant angular velocity (CAV) storage, where the

information density is greater on the inner tracks that the outer tracks.

While the use of CLV recording increases the storage capacity of CD-ROM

disks, it lengthens access time, since the disk must change speeds as it

moves to different positions on the disk radius.  The strategy for locating

a recorded data block is also more complicated than with constant angular

velocity recording technology.

As a result the average access time of CD-ROM drives is between 400 ms

and 1 s, and the maximum data transfer rate to the host computer is 150

kilobytes per second.  These rates are approximately an order of magnitude

slower than magnetic disks.  Therefore, the access mechanisms to disk

directories and data must be customized for CD-ROM in order to provide

acceptable performance.

The development of a standard logical format for recording data files on

CD-ROM disks was taken on by a group of CD-ROM applications developers,

hardware vendors and computer vendors.  The proposed standard resulting

from this initial effort was called the High Sierra format.  This format

was submitted to the International Standards Organization (ISO) for

consideration and approved on October 5, 1987.  It is now referred to as

the ISO-9660 standard.  The format used for the early PDS CD-ROM disks is

dated May 28, 1986, and differs slightly from the final ISO standard.

Microsoft and other CD-ROM software developers have committed to supporting

this preliminary version of the format in later software releases.

However, software developed by other vendors to support the ISO format (DEC

for example) will not support the format of these early disks.

Chapter 3

IMDISP INSTALLATION

IMDISP requires an IBM PC, XT, AT or 100 percent compatible with 512K of

base memory.  It supports several graphics display devices, including the

Enhanced Graphics Adapter (EGA), the Video Graphics Array (VGA), the

Professional Graphics Adapter (PGA), and the Color Graphics Adapter (CGA).

The hardware environment in which the program has been developed and tested

includes and AT class computer, deluxe or enhanced EGA board with 256K of

memory, selected VGA boards with 512K or 1MB memory, and MultiSync (tm) or

multiscan monitor.

3.1 PROGRAM SETUP

The IMDISP program can be run from a floppy or hard disk.  To use the

program from a floppy disk, insert the program disk in the A: or B: drive

and type the MS-DOS command: "A:" or "B:" to set the floppy disk as the

default drive.  To use the program from hard disk first create a directory

to hold the IMDISP files.  Use the MS-DOS command "MKDIR \IMDISP" to create

a directory to hold the programs.  Use the "CHDIR \IMDISP" command to make

IMDISP the default directory.  Insert the distribution floppy disk in

floppy disk drive A:.  Now type "COPY A:*.*" to copy the files from the

distribution disk to your hard disk.  You should now be able to run all

examples from that directory on your hard disk.  If you wish to use IMDISP

from other directories, you need to include the IMDISP directory in your

MS-DOS "PATH" command.  This command provides MS-DOS with a list of

directories to search when looking for a program file to run.  If you wish

to run IMDISP from different directories, the \IMDISP subdirectory should

be added to the PATH command in your AUTOEXEC.BAT file.  A sample path

command would look like this:

PATH=C:\DOS;C:\UTIL;C:\WORDSTAR;C:\IMDISP

You may want to add a line to your AUTOEXEC.BAT file which will allow

you to abort the display program while running batch files or displaying

large image files.  If the command "BREAK=ON" is placed in CONFIG.SYS, you

will (sometimes) be able to terminate the IMDISP program by typing the

control and break keys simultaneously.  Different computer systems offer

varying response to the control-break command, and the use of this command

may effect other programs you use.

You can set IMDISP to start up in the correct display mode for your

video system by specifying the environment variable IMDISP in your

AUTOEXEC.BAT file.  A sample command to do this would look like this:

SET IMDISP=ORCHID800

if you have an Orchid ProDesigner super VGA board with 512K of video

memory.  See section 3.3 for a description of all the possible supported

video modes.

IMDISP also builds various scratch files for use when browsing images or

executing DOS functions.  By default, they go in the root directory on

drive C:.  If you wish to have them written somewhere else, you may do that

by specifying the desired directory in the environment variable IMBROWSE.

A sample command to do this would look like this:

SET IMBROWSE=D:\TEMP\

This will make IMDISP write its scratch files in the subdirectory TEMP on

drive D:.  Note that the final backslash (\) is required.  We recommend

that the drive specified by IMBROWSE have at least 512K of free space - the

scratch file written when using any of the DOS system calls can be as large

as that.

Also, if you have a collection of color palettes in a separate

subdirectory, you can specify this directory as being the default palette

directory.  For example:

SET IMPALETTE=D:\IMAGES\PALETTES\

will tell IMDISP to look in the D:\IMAGES\PALETTES directory first when

loading in a color palette with the PAL LOAD command.

IMDISP can use extended memory, managed by an XMS memory driver like

HIMEM.SYS or QEMM, for its refresh buffer.  If no extended memory is

available, IMDISP will not set up a refresh buffer by default, but will

allow the user to open a disk file to be used for refreshing the screen.

In this case, the environment variable IMREFRESH defines where the refresh

buffer is to go, so

SET IMREFRESH=G:\

tells IMDISP to put the refresh buffer in the root directory of drive G:.

Note that the refresh buffer can be quite large - a 1024x768 display will

require 768K of disk space (or XMS memory) to hold the entire buffer.

3.2 CD-ROM SETUP

To use the IMDISP program with PDS CD-ROM disks you will need a CD-ROM

reader, interface board, and software provided by the vendor which will

make your CD Reader look like a disk drive to your PC.  The details of

setting up your hardware and software configuration are beyond the scope of

this manual, however several aspects of setup will be reviewed.

There are many potential pitfalls in setting up your hardware and

interface board.  Read the directions carefully, and don't assume that

"default" switch settings are correct.  Check all switches to see that they

correspond to the recommended settings.

To work properly with the batch command and label files provided with

IMDISP, your CD-ROM drive should be set up to be drive letter "L:".  Most

vendor software allows you to select the drive letter which will be

assigned to the CD-ROM reader.

3.2.1 Microsoft Extensions

If you are using the Microsoft Extensions you will load a CD-ROM device

driver as part of your CONFIG.SYS file (or two drivers if using the Philips

or DEC readers) with a line like this:

DEVICE=HITACHI.SYS /D:CDROM1 /N:1

This command assumes that the driver file "HITACHI.SYS"is located in your

ROOT directory.  The /D:CDROM1 switch assigns a logical name CDROM1 to the

drive.  A name must be supplied, and is used to identify this device, in

case more than one CD-ROM drive is being used on your system.  The name

should not be the same as the name of any file stored on your system.  An

attempt to open a file with the same name as the device driver will result

in the device driver being opened, not the file.  The /N:1 switch indicates

that this is drive 1.  This switch is used since some CD-ROM interface

cards support multiple drives.

Other commands which should be included in CONFIG.SYS if you are using a

CD-ROM drive with the Microsoft Extensions are:

LASTDRIVE=Z

FILES=30

BUFFERS=50

The LASTDRIVE command tells MS-DOS to allow for enough drive letters to

support the CD-ROM drive, plus any other drives on your system.  Using

LASTDRIVE=L should also work in most circumstances.  The files and buffers

specifications provide sufficient system work area to support CD-ROM

operations.

You must also execute a program called MSCDEX.EXE which allows access to

the CD-ROM drive as if it were a disk drive on your system.  This command

can be put in your AUTOEXEC.BAT file so that it is automatically run

whenever you start-up your computer.  The command format is:

MSCDEX /D:CDROM1 /L:L /M:20 /V /E

Several MSCDEX command switches are illustrated.  The /L:L command

assigns the drive name L: to the CD-ROM drive.  The /D switch gives the

device name of the CD-ROM drive (which must be the same name used in the

device name parameter of  the "DEVICE=CDROM.SYS" command in the CONFIG.SYS

file.  The /M:20 switch assigns twenty 2K blocks of memory as a buffer for

use with CD-ROM data.  The /V switch provides verbose messages when the

installation program is run, and /E tells the program to use expanded

memory for the cache area.

3.2.2 Other Driver Software

Some vendors (Reference Technology, TMS) offer CD-ROM software which is

comparable to the Microsoft Extensions.  Our experience with these software

packages is that they modify the internals of MS-DOS and may cause

unpredictable results with your other PC software.  They often do not

support CD-ROM applications designed to work with the MS-DOS extensions for

CD-ROM and cannot be used with the IMDISP FILE prompt mode.

3.2.3 Problems with CD-ROM Software

The PDS CD-ROM disks utilize a feature of the CD-ROM format standard

which provides extended attribute records to define the physical

characteristics of data files.  These records will be used by VAX and other

minicomputer systems where the operating system allows a variety of record

formats (fixed, variable, stream).  Many of the earlier implementations of

High Sierra software failed to recognize these records, which are placed at

the beginning of a file's data area.  If your CD-ROM software was developed

prior to April 1987, it may interpret these records as part of the data

file, causing the first 2K bytes of each file to appear as meaningless

binary data.  Users with this problem should contact their hardware or

software vendors for updated versions of the CD-ROM software.  The IMDISP

program has been patched to recognize and skip over these records on the

PDS CD-ROM disks, but these patches may not support other CD-ROM disks with

extended attribute records.

3.3 DISPLAY DEVICE CHARACTERISTICS

IMDISP automatically finds out which display devices are available (PGA,

EGA, or CGA) and uses the one available.  The Enhanced Graphics Adapter

must have at least 128K of memory to work properly.  Use of the Color

Graphics Adapter is discouraged because the pixels have no gray levels,

only black or white.

The display coordinates start at (1,1) in the upper left corner of the

screen; the line direction is down and the sample direction is to the

right.

3.3.1 Enhanced Graphics Adapter (EGA)

The EGA is a bit-mapped display device providing a resolution of 350

lines by 640 samples of 4-bits each.  It produces a digital video signal

for each primary color (red, green and blue), The video signal for each

color can be set to one of four levels, roughly equivalent to off, low,

medium and high.  Thus the number of possible colors (color palette) is 64

(4^3).  The following table illustrates the colors created by some of the

different combinations of red, green and blue.

Red=   off, Green=   off, Blue=   off  results in Black

Red=   low, Green=   off, Blue=   off  results in Dark Red

Red=medium, Green=   off, Blue=   off  results in Light Red

Red=  high, Green=   off, Blue=   off  results in Bright Red

Red=   low, Green=   low, Blue=   low  results in Dark Gray

Red=medium, Green=   low, Blue=   off  results in Brown

Red=medium, Green=medium, Blue=   off  results in Yellow

Red=medium, Green=medium, Blue=medium  results in Light Gray

Red=  high, Green=   low, Blue=   off  results in Orange

Red=  high, Green=  high, Blue=  high  results in White

Table 3-1: Sample EGA Color Palette Settings

The EGA display is limited to only 16 colors out of the 64 possible,

because only 4-bits are used to store each pixel value in memory.  This

4-bit pixel value points to an entry in the color table which represents

one of the 64 possible combinations of red, green and blue which can be

displayed.

Many of the newer EGA boards are capable of displaying additional lines

and samples when using a MultiSync (tm) or Multiscan monitor.  The standard

EGA uses a 16 Mhz crystal oscillator (something like a clock which

regulates the display speed), which limits the video output rate to the

equivalent of 640 pixels by 350 lines.  The EGA circuitry allows for a

faster crystal oscillator to be added to the board (24 Mhz and beyond)

allowing about 25 percent more lines to be displayed (480 lines instead of

350).  A method for adding this capability to an existing EGA board (for

about $10 worth of parts) is described in the September 16, 1986 issue of

PC Magazine.

This mode requires that a MultiSync (tm) or multiscan monitor be

attached to the computer, and there is no way for the program to

automatically detect the presence of the special monitor.  Therefore this

display mode is invoked by specifying an MS-DOS "ENVIRONMENT" variable.

This is done by issuing an MS-DOS "SET" command at the MS-DOS prompt as

follows:

SET IMDISP=EGA480

This command can also be put in your AUTOEXEC.BAT file using a text

editor, so that you need not invoke it each time you run IMDISP.  It should

have no effect on other MS-DOS system operations.  There is a chance that

adding this variable will exceed the size reserved by MS-DOS for environment

variables.  If so, you should consult your MS-DOS manual to increase the

environment size on your system.  The environment variable can be removed

with the MS-DOS command:

SET IMDISP=

where a carriage return is typed immediately after the equal sign.  The 480

line mode requires an additional 83K of memory for the refresh buffer.  If

you try this mode and receive the message "Insufficient memory for line

buffer", it is probably because you have a 512K memory machine, or have

memory resident programs operating (like Sidekick or Superkey) which reduce

the available memory to less than about 400K.  You will need to use the 350

line mode or remove some memory resident programs to operate in 480 line

mode.

3.3.2 Video Graphics Array (VGA)

This version (7.9) of the IMDISP program includes support for the IBM

Video Graphics Array (VGA) display device.  The VGA produces an analog

display with several new color modes.  Of particular interest to IMDISP

users are the 640 x 480 line mode with 16 displayable colors (or gray

levels).  The 320 x 200 line mode with 256 displayable colors (or 64 gray

levels) has been tested but produces very blocky pixels.  This version of

IMDISP also supports a number super VGA graphic boards (see below).

To set the VGA display mode you must set a MS-DOS environment variable

prior to executing IMDISP.

To set the environment variable use the MS-DOS command:

SET IMDISP=VGA

This will put the display in 640 sample by 480 line mode, with 16 gray

levels or colors displayable from a palette of 256K.  The display is

initialized with a gray scale palette, but a pseudocolor palette can be

invoked with the IMDISP command "PAL PS 0".  The "PALETTE EDIT" command

will allow you to step forward or backward through the 64 available shades

for each primary color.  See the next section of the IMDISP manual for more

information on the palette edit function.

Users may experiment with the 320 x 200 VGA mode by using

SET IMDISP=VGA320

The pixels are rather large in this mode, but there are enough colors to

display images nicely.

3.3.3 Professional Graphics Adapter (PGA)

The PGA is a special graphics board developed by IBM for use with

CAD/CAM applications on the IBM PC.  Because its architecture is

incompatible with the CGA and EGA boards, it has not been well received by

software developers.  The PGA provides an analog video signal with a

display resolution of 480 lines by 640 samples of 8-bits each.  It provides

16 intensities for each primary color, resulting in a palette of 4,096

colors (16^3).  The color table allows 256 colors to be displayed

simultaneously, however only 16 gray levels can be selected.

Use the command:

SET IMDISP=PGA

at the MS-DOS prompt to force IMDISP to go into PGA display mode.

3.3.4 Color Graphics Adapter (CGA)

The CGA display is a bit-mapped graphics device with a resolution of 200

lines by 640 samples of 1-bit each.  It supports only 2 colors in this

display mode, black and white.  This display can be used to get a general

idea of the contents of a digital image, or for displaying one bit images

(graphics for example) but is not recommended for use with the PDS CD-ROM

images.

To force a multi-function display card into CGA mode, use:

SET IMDISP=CGA

3.3.5 Super VGA Display Boards

Several high resolution display boards are supported by this version of

IMDISP.   Most of them support resolutions with 256 colors or 64 grayscale

levels.  Select one from the following list if it matches your display

board:

SET IMDISP=     Resolution     Board Name

ATI640      640x480x256    ATI VGA Wonder Board (512K)

ATI800      800x600x256    ATI VGA Wonder Board (512K)

ATI1024     1024x768x16    ATI VGA Wonder Board (512K)

EVGA512     512x480x256    Everex EV-673 Board  (256K)

EVGA640     640x400x256    Everex EV-673 Board  (256K)

ORCHID      640x480x256    Orchid ProDesigner+ Board (512K)

ORCHID800   800x600x256    Orchid ProDesigner+ Board (512K)

ORCHID1024 1024x768x256    Orchid ProDesigner+ Board (1MB)

PARADISE    640x480x256    Paradise VGA Board (400 lines if the

board has 256K of memory)

TRIDENT     640x480x256    Trident-based SVGA boards (512K)

VESA       1024x768x256    VESA SVGA Interface (1MB) (not fully

implemented)

3.4 MULTIPLE BUFFERS

Multiple copies of images (or the entire screen) can now be copied and

stored away into buffers, limited by the available memory.  If you don't

have enough memory, the buffers can still be used, but as virtual files

(see HINTS & TIPS ON CONSERVING MEMORY, down below).  Using the buffers,

images can now be merged together to create interesting effects.  You can

add or subtract images from each other.  You can store an image away into a

buffer before applying a filter, and if you didn't like the result, then

retrieve it back from the buffer, effectively creating an UNDO feature.

You can create a "face-on-Venus image" by merging the face-on-Mars image

with a Magellan image.

There are 26 buffers available in IMDISP, limited by memory or disk space.

The buffers are referenced by name as a single letter, one for each letter

of the alphabet (A-Z).

3.5 HINTS & TIPS ON CONSERVING MEMORY

If you have no extended memory, or don't have enough extended memory, don't

despair.  You can still use the buffers if you use the following tips to

help you maximize the use of the memory

o IMDISP will automatically create a refresh buffer in extended memory

whose size is the size of the screen.  You can disable the refresh

buffer and this memory will be freed up for buffer use.  To disable

the refresh buffer, type:

SET REFRESH OFF

The refresh buffer can be enabled with

SET REFRESH ON

o IMDISP will also attempt to load an entire image into extended

memory whenever you do the FILE command.  You can have IMDISP not do

this by using the NOMEMORY option:

FILE IO.IMG NOMEMORY

o If you are using a Super VGA graphics card in its highest

resolution, then if you switch to a lower screen resolution, this

will generally cause the buffers to be smaller.  For example, if you

using an Orchid Pro Designer II card in its highest resolution

(1024x768), then you can switch to the 800x600 or 640x480 screen

resolutions:

SET DIS ORCHID800

SET DIS ORCHID

o Subsample the image so that it doesn't take up the entire screen,

which then can be saved to smaller buffers:

DISP SUB 3

o Use the FILE option on the COPY or MERGE commands to store buffers

into a file instead of extended memory.

o Erase any unused buffers:

ERASE A

3.6 THE HELP FILE

The HELP command in IMDISP uses a plain text file, called IMDISP.HLP,

which contains brief summaries of each command and its options.  This file

can be modified as you wish with any text editor or word processor capable

of saving files in plain ASCII format.

IMDISP looks in several places to try to find the help file when the

program starts.  First, the DOS environment is searched for the environment

variable IMHELP.  If IMHELP is found, and contains a file name, the program

will attempt to open that file (so you can call the help file anything you

want).  If IMHELP contains only the name of a subdirectory, the program

will attempt to open IMDISP.HLP in that directory.  If that fails, or if

IMHELP is not set, the program then looks in the current directory for

IMDISP.HLP.  If IMDISP.HLP isn't found in the current directory, and if you

are using MS-DOS v3.3 or later, the program tries to open the help file in

the directory where the IMDISP.EXE program file is located.  If all these

fail, the program reports that it can't open the help file and continues.

For example, to use a file called MYHELP.IMD, located in the

subdirectory C:\HELPFILES, instead of using the default file IMDISP.HLP,

issue the command

SET IMHELP=C:\HELPFILES\MYHELP.IMD

either at the DOS prompt or in your AUTOEXEC.BAT file.  To use the file

IMDISP.HLP, but to allow the program to find it in the subdirectory

C:\HELPFILES, issue the command

SET IMHELP=C:\HELPFILES

or

SET IMHELP=C:\HELPFILES\

Chapter 4

IMDISP OPERATION

IMDISP can be used to display images up to several thousand lines and

samples with a variety of pixel formats.  These formats include bit (1

bit), nibble (4 bits), byte (8 bits), integer (16 bits) or long integer (32

bits).  Integer pixels may be byte-swapped, which means that the sign and

most significant 7 bits are in the rightmost byte, and the least

significant 8 bits are in the leftmost byte.  This is the convention used

by both the IBM PC and VAX computer hardware families.  IBM mainframes and

68000 series computers (Macintosh and Sun, for example) use non-byte-

swapped integers.

Program interaction with the user is via commands typed in response to

the "COMMAND:" prompt.  Most interaction is performed on the graphic

display screen, and program messages are printed over any image display

currently on the screen.  The REFRESH command can be used to restore an

image after other commands have written text over the display.  The ERASE

command can be used to clear away the clutter left by program status

messages and previously displayed images.  The screen is not automatically

erased after each operation in order that multiple images can be displayed

simultaneously, or so a histogram can be placed on the screen with the

image.

IMDISP is invoked by typing "IMDISP" or "IMDISP filename", where

filename is the name of an image file to be opened for processing at

program start-up.  The filename may include a disk drive and path name

specification.

The program will blank the display screen and display a welcome logo,

then the prompt "COMMAND:" will appear in the lower left portion of the

screen.  If a filename is included in the command invocation, the welcome

message is not displayed and the screen will display the COMMAND: prompt.

Typing "HELP" at the command prompt will provide a list of IMDISP

commands.  The most frequently used commands are "FILE" to open a file for

processing, and "DISPLAY" to display an image once the file has been

opened.

Previous commands can be recalled for editing, so you don't have to type

them over again.  Use the up arrow (or cursor up) key to recall old

commands.  The left and right arrow (cursor) keys move you back and forth

in the command.  Make whatever changes you want, then press the Enter key

to execute the command.

4.1 COMMAND SYNTAX

The command line syntax is of the form:

COMMAND KEYWORD1 = VALUE1 KEYWORD2=VALUE2 KEYWORD3 VALUE3 ...

The command line may be typed in either upper or lower case.  The

command and the keyword names may be abbreviated to 3 characters in most

cases.  Any number of spaces may be inserted between words, and the keyword

and value may be separated by a space or an equals sign.  Some keywords do

not require a value.  All keywords are optional and have default values.

4.2 COMMAND DESCRIPTION

Table 4-1 presents a summary of IMDISP commands.  They are separated

into 3 groups, file manipulation commands; display commands and program

control commands.  Only the capitalized letters are required to specify a

command.

4.3 OPERATING MODES

Several of the commands invoke special program operating modes.  These

include the FILE command when issued without a filename argument and the

CURSOR, PROFILE and PALETTE EDIT commands.  The FILE mode displays a list

of files in the current directory on the screen for selection.  It also

contains several subcommands for controlling the display of files on the

menu screen.  In the CURSOR, PROFILE and PALETTE EDIT commands the cursor

keys (arrow keys) on the numeric keypad are used in conjunction with other

keys to control program operation.  These modes are exited by typing either

a period '.' or carriage return.

File oriented commands:

CD or CHDIR .  .  .  .  to change the default directory

DIRECTORY   .  .  .  .  to perform the MS-DOS directory command

FILE        .  .  .  .  to specify the name of the image

LABEL       .  .  .  .  to display the image labels

LOG         .  .  .  .  to save (log) commands to a text file

SAVE        .  .  .  .  to save the image display to a file

TYPE        .  .  .  .  to perform the MS-DOS type command

Display commands:

BROWSE      .  .  .  .  to display a group of images

CURSOR      .  .  .  .  to move the cursor around

DISPLAY     .  .  .  .  to display the image

ERASE       .  .  .  .  to erase the display

HISTOGRAM   .  .  .  .  to display the histogram of the image

OVERLAY     .  .  .  .  to place an overlay on the image

PALETTE     .  .  .  .  to adjust the palette for the display

PERSPECTIVE .  .  .  .  to plot a pseudo-perspective of the image

PLOT        .  .  .  .  to plot image lines and spectra on the display

PROFILE     .  .  .  .  to plot a profile of the image

REFRESH     .  .  .  .  to refresh the image plane

SET         .  .  .  .  to set display options

TEXT        .  .  .  .  to draw text on the image

Image processing commands:

BRIGHTEN    .  .  .  .  to brighten the image

COPY        .  .  .  .  to copy images to/from memory buffers

DARKEN      .  .  .  .  to darken the image

ENHANCE     .  .  .  .  to apply various filters to the image:

BAALKE

CONVOLUTION

EDGE

HIGH

HISTOGRAM

LOG

MEAN

MEDIAN

POWER

STAIR

SMEAR

UNSHARP

MASK        .  .  .  .  to mask off pixels in the image

MERGE       .  .  .  .  to combine image and memory buffers

NEG         .  .  .  .  to invert the color palette

PALETTE     .  .  .  .  to adjust the palette for the display

ROTATE      .  .  .  .  to rotate the image

SLANT       .  .  .  .  to slant the image to the right or left

STRETCH     .  .  .  .  to do a linear gray scale stretch

WINDOW      .  .  .  .  to mark off a subimage on the screen

Program control commands:

BATCH       .  .  .  .  to execute a batch command file

EXIT or QUIT.  .  .  .  to exit from the program

HELP        .  .  .  .  Display help information

MENU        .  .  .  .  to select images from a menu file

SYSTEM      .  .  .  .  execute an MS-DOS command

Table 4-1: IMDISP Command Summary

4.4 ERROR HANDLING

If an invalid command is issued at the COMMAND: prompt the program will

beep and return to the COMMAND: prompt.  In handling command parameters the

program takes action on those parameters that are recognized, but ignores

invalid parameters.

It is also possible for certain system errors to be encountered which

cause the program to abort leaving your computer in graphics mode.  If you

are using certain utilities which reset the default text mode screen colors

(like the Norton Utilities screen attributes command) you may not be able

to see what is being printed on the screen.  The simplest approach is to

perform a warm boot, by typing the Ctrl, Alt, Del keys simultaneously.

Alternately, you can often type IMDISP then EXIT to reset the computer to

text mode.  Errors in processing image files on CD-ROM, hard disk or floppy

disks can cause these aborts.

4.5 COMMAND REFERENCE

This section lists all IMDISP commands alphabetically and describes

their function, parameters which control  command operation, and provides

examples of command use, In the command and parameter description the

following conventions are used:

filename    represents the name of an MS-DOS file, and may include an

optional drive identifier and path specification.

n           represents an integer value.

4.5.1 BATCH

The BATCH command reads commands from the specified batch command file.

If no file name is specified the current directory is searched for a file

named BATCH.CMD.  The CURSOR, PROFILE and PALETTE EDIT commands should not

be used within batch command files since they require interactive inputs

from the keyboard.  Batch files may be nested.

The BATCH command may take the following argument:

filename    filename of batch command file.

A batch file is simply a text file containing a list of commands, with

each command terminated by a carriage return.  You can add comments to a

batch file by putting a semi-colon on the line.  Everything from the

semi-colon to the end of the line is ignored.  If the command file is

created with a word processor, the program should be used in the

non-document mode, to assure that carriage returns are embedded between

command lines.  An example of creating a batch file to display the PDS logo

image follows.

At the MS-DOS command level type:

COPY CON TEST.CMD

FILE LOGO.IMG

DISP ZOOM 4

DISP ZOOM 2

DISP

DISP SUB 2

EXIT

Now run IMDISP and type "BATCH TEST.CMD" at the "COMMAND:" prompt.  Make

sure the file "LOGO.IMG" is in your current directory.  The PDS logo should

be displayed on the screen in several different sizes.

4.5.2 BRIGHTEN

The BRIGHTEN command adds a constant value to all pixels in the

displayed image.  If no value is specified, the default is 10% of the total

range of DN values (i.e., 2 will be added to all pixels in 16-color modes,

26 will be added in 256 color modes).

Examples:

BRIGHTEN

BRIGHTEN 50

4.5.3 BROWSE

The BROWSE command will display all images in a directory one after

another.  An optional directory mask can be supplied to display only

selected files ("BROWSE *.IMG").  The command creates a list of files to be

displayed, then creates and executes a batch command file to display those

images.  After each image is displayed, the filename is displayed at the

top of the image.  A BROWSE session can be interrupted by pressing

control-s, then restarted by pressing control-q TWICE.  A session can be

halted by typing any other key during the display.  It can then be

restarted (from the beginning) with the command "BROWSE C:BATCH.CMD".  The

default file name for the batch file created by the BROWSE command is

"C:BATCH.CMD".  This name can be changed using the "SET BROWSE fname"

command where fname can consist of a drive letter and filename, which

should be terminated  with the extension ".CMD".  It can also be set from

DOS or in your AUTOEXEC.BAT file by using the environment variable

IMBROWSE.  For example, the DOS command

SET IMBROWSE=D:\TEMP\BROWSE.CMD

will make the BROWSE command file in the subdirectory TEMP on drive D:.

The BROWSE command has one argument and several optional keywords.

fname         optional file selection mask (must immediately follow the

command BROWSE).

Examples: *.img, D:\IMAGES\*.IMG, L:*.BRS.

SIZe n        allows the display of multiple images on the screen.  For

example, on a 640x480 VGA display, six 200x200 images can

be displayed adjacent to each other.

SUBsample n   used in conjunction with the SIZE command, to scale each

image to fit in the SIZE specification.  Same as SUB

option used with the DISPLAY command.

NOLabel       inhibits writing the file name label at the top of each

image as it is displayed.  The default is that labels are

enabled.

DNLow DNHigh  sets the DN range for all images (see SET).

PAUse n       pauses display for n seconds when the screen gets full.

ALL           searches all subdirectories in addition to the current

directory, looking for files matching the mask.

AUToset n     sets DNLO and DNHI to n% of the full range, based on the

image histogram.  The default is 5%.

SELect        interactively selects files as they are being displayed

and writes the filenames out to a "select" file.  The

default select file is "C:\IMDISP.SEL" and this can be

changed by using the SET SELECT command.

FILe fname    select files to browse from a file instead of using a

mask.  The file will contain a list of files, and the list

can have wildcarded names.  The filename can also be the

IMDISP.SEL file created earlier with the SELect option.

BUFFERS       browses through all the memory buffers which have been

created.

EXAMPLES:

BROWSE *.* SIZE 200 SUBSAMPLE 4

will display all the files in the current directory, subsampling each by 4

and displaying them in a 200 x 200 window.  The command:

BROWSE *.IBG SIZE 100 SUB 2 PAUSE ALL

when used with the Voyager browse images will display all of the browse

images on the screen (including the subdirectories), pausing each time the

screen fills up with images.

BRO *.* SIZE 200 ALL SELECT NOLABEL

will allow the selection of individual images and

BRO FILE C:\IMDISP.SEL SIZE 200

will redisplay the images that you just selected.

BROWSE BUFFERS PAUSE

displays the contents of each buffer, pausing in between each display.

BRO BUFFERS SIZE 100 SUB 4

displays a small version of each buffer.

*** NOTE ***

BROWSE does not distinguish between image and non-image files, and it is

up to the user to provide the appropriate directory mask to select ONLY

images.  If a message "Image does not have proper label" comes up on the

screen during BROWSE, it is probably because a non-image file is in the

directory you are browsing.   You can type carriage returns several times

to get back to the "COMMAND:" prompt, and the browse will continue.

If you are using BROWSE on a computer without a hard disk, you will have

to use the SET BROWSE command to change the default browse file name to a

writable disk drive,  for example: "SET BROWSE A:BROWSE.CMD".  If you are

using BROWSE to view files on a CD-ROM disk, be sure to include the drive

letter of a writable disk  drive if you specify a BROWSE file name.  The

text editor to produce your own custom command files.

4.5.4 CD or CHDIR

The CD or CHDIR command is used just as it is at the MS-DOS command

level to change the current default directory.

The  argument  of the CD command is the  path  name specification of the

new default directory.  For example, "CD \IMDISP\IMAGES" would make

\IMDISP\IMAGES the current directory if it exists.  To change the current

default drive use the "SYSTEM" command ("SYSTEM L:"  for example) or the

"FILE" command.  If there is not enough memory available for the system

command to be executed an error message is displayed.

4.5.5 COPY

The COPY command will attempt to copy the image last displayed with the

DISPLAY command, the screen or another buffer to a destination buffer.

IMDISP will attempt to put the buffer into extended memory.  An FILE option

is provided to allow the buffer to be stored on the disk, rather than in

memory.

COPY  buffer [FILe]

COPY  TO buffer [FILe]

COPY  source TO buffer [FILe]

where buffer = A-Z

where source = A-Z or SCReen

Examples:

COPY A                Copies the last displayed image to buffer A

COPY A TO B           Copies buffer A to buffer B

COPY A TO B FILE      Copies buffer A to buffer B, buffer B is actually

stored on disk, instead of in memory

COPY SCREEN TO Z      Copies the entire screen to buffer Z

4.5.6 CURSOR

The CURSOR command is used to select a point in a displayed image for

subsequent display commands.  When first invoked, it places a small cursor

symbol at the center of the screen.  Depending on the current contents of

the screen, the cursor may be hard to see.  Moving it with the arrow keys

on the numeric keypad will help you locate it.  If you have a mouse, it

will also move the cursor around.

Whenever the cursor is moved, the current line and sample location of

the cursor and the DN value of the pixel at that point is printed in the

lower right portion of the screen.  The cursor will move 16 pixels in the

selected direction unless the movement value is adjusted using the + or

keys.

Numeric Keypad Keys

�����������������������������Ŀ

�  7   �  8   �  9    � PrtSc �

� Home � Up   � PgUp  �       �

�����������������������������Ĵ

�  4   �  5   �  6    �  -    �

� Left �      � Right �       �

�����������������������������Ĵ

�  1   �  2   �  3    �  +    �

� End  � Down � PgDn  �       �

�������������������������������

Action of Keypad Keys:

7 = Move up and left.

8 = Move up.

9 = Move up and right.

4 = Move left.

5 = Not active.

6 = Move right.

1 = Move down and left.

2 = Move down.

3 = Move down and right.

- = Decrement cursor movement value.

+ = Increment cursor movement value.

Table 4-2: IMDISP Cursor Keys

Cursor mode is exited by typing a "." (period) or carriage return or by

tapping the left button on the mouse.  The cursor position is retained for

future use in DISPLAY commands which use the CENTER option.  It is very

useful with large images where the entire image may be displayed using the

subsample option, then a point selected for subsequent display at full

resolution.

You can specify the starting position of the cursor with the subcommands

DSL (Display Starting Line) and DSS (Display Starting Sample).  Otherwise,

the cursor starts off at the position where it was left last.

Example: Type "FILE MONTAGE.IMG" followed by "DISP" to display the

MONTAGE.IMG file.  Type "CUR" to enter the cursor mode, then move the

cursor to the center of the Uranus image in the upper left corner of the

image using the up and left arrow keys.  Now type "." to exit cursor mode,

then type "DISP ZOOM 4 CENTER" which will display Uranus as a large grainy

ball on the screen.

4.5.7 DARKEN

The DARKEN command subtracts a constant value from all pixels in the

displayed image.  If no value is specified, the default is 10% of the total

range of DN values (i.e., 2 will be subtracted from all pixels in 16-color

modes, 26 will be subtracted in 256 color modes).

Examples:

DARKEN

DAR 100

4.5.8 DIR

The DIR command is used just as the MS-DOS dir command.  Its argument is

passed to MS-DOS for execution.  At the end of the directory listing the

screen will display "Type carriage return to continue:" which will return

to the IMDISP command mode.  If there is not enough memory available for

the system command to be executed an error message is displayed.

4.5.9 DISPLAY

The DISPLAY command reads an image file and displays it on the the

screen.  If the image is larger than the display screen, the default mode

is to display as much of the upper left portion of the image as will fit on

the screen.  The SUBSAMPLE keyword can be used to select every 'n' lines

and samples so that representations of very large images may be displayed.

The ZOOM keyword can be used to enlarge a portion of an image on the

display screen.  Keywords can be specified to begin displaying at any line

and sample in the image with the SL (starting line) and SS (starting

sample).  If only a limited number of lines and samples are to be displayed

the NL (number of lines) and NS (number of samples) keywords can be used.

The UP, DOWN, LEFT and RIGHT keywords can be used to display different

portions of the image, relative to the currently displayed portion.  The

display can also placed at any point on the screen using the DSL (display

starting line) and DSS (display starting sample) keywords.  The FLIP

keyword displays the image from the bottom of the screen to the top, rather

than the other way around.

DISPLAY takes the following keywords :

buffer       displays the image in the named memory buffer

SUBsample n  a positive integral subsampling factor for displaying

images larger than the size of the screen.  SUB=2 would

display an image at half the resolution.

If the image lines and samples look like this:

sample values

line 1      1 2 3 4 5

line 2      6 7 8 9 10

line 3      11 12 13 14 15

Then the command DISP SUB 2 would produce:

sample values

line 1      1 3 5

line 2      11 13 15

ZOOM n       a positive integral zoom factor for looking at part of an

image close up.  The zoom command replicates pixels by the

integer value specified.  If the image lines and samples

look like this:

sample values

line 1      1 2 3

line 2      4 5 6

line 3      7 8 9

Then the command ZOOM 2 would produce the following:

sample values

line 1      1 1 2 2 3 3

line 2      1 1 2 2 3 3

line 3      4 4 5 5 6 6

line 4      4 4 5 5 6 6

line 5      7 7 8 8 9 9

line 6      7 7 8 8 9 9

CENTER       center the display around current cursor position.   This

command is very useful when displaying large images or when

zooming in on selected areas.  Should be used after using

the CURSOR command.

NL n

NS n         number of lines or samples from the image file to display

SL n

SS n         starting line or sample within the image file to display

UP n

DOWN n       adjusts the starting line up or down from the previous

value, thereby moving  the display window up or down in the

image.  The default amount to move up or down is the size

of the display.

LEFT n

RIGHT n      adjusts the starting sample left or right from the previous

value, thereby moving the display window left or right in

the image

DSL n

DSS n        start the image display at the specified line and sample on

the display screen.

FLIP         flips the image top to bottom, so the image is displayed

from the bottom of the screen to the top.

NOPrompt     displays the image without display the command line prompt

(COMMAND:) at the bottom of the screen.  This is to

accommodate screen captures of IMDISP displays.

AUToset n    sets DNLO and DNHI to n% of the full range, based on the

histogram.  The default is 5%.

NOMemory     tells IMDISP not to attempt to load the image into extended

memory as it displays the image.

SOUrce       used to display the source file of a browse image.  The

source file is specified with the SOURCE_FILE_NAME and

SAMPLING_FACTOR keywords in the label.  The source file is

centered on the current cursor position in the browse file.

Does not work with Voyager or Viking compressed images.

Examples: The user wants to display an entire Voyager image (800 x 800)

on the display screen then display a portion of the image at full

resolution.  Type "FILE MIRANDA1.LBL", then "DISP SUB 2" to display the

entire image at half resolution, Use the CURSOR command to move the cursor

to the center of an area of interest.  Exit the cursor mode by typing ".",

then type "DISP CENT" to display the selected portion of the image at full

resolution.

The user wishes to place 2 images which are each 800 lines by 800

samples on the screen next to each other.  The first file is opened with

"FILE IMAGE1.IMG".  The "DISPLAY SUB 4" command is given to display every

4th line and sample of the image to create a 200 x 200 display.  Next the

"FILE IMAGE2.IMG" command is given.  Now a "DISPLAY SUB 4 DSS 201" is given

to place the second image on the screen starting at display sample position

201, next to the first image.

Examples:

DISP SUB 2

DISP SUB 3 DSS 200

DISP SUB 4 DSL 300

DISP CEN ZOOM 2              (Use CUR first to select a center)

DISP A                       Displays buffer A

DISP A SUB 2                 Displays buffer A, subsampled twice

DISP A CEN ZOOM 3            Zooms in on the center of buffer A

4.5.10 ENHANCE

The command ENHANCE applies a number of image processing filters to the

image.  It can either be applied to the entire image (from the input file)

or simply to the displayed portion of the image.

ENHANCE takes the following subcommands:

SCReen         applies the filter to the entire screen.  If this

option is not selected, then the filter will be applied

to the image last displayed with the DISPLAY command.

BAAlke         applies an enhancement filter developed by Ron Baalke.

CONvolution n  applies a convolution filter to the image which has the

effect of detecting the edges.  A 3x3 mask is used.

There are two variations of this filter used (n = 0 or

1).  Integer arithmetic is used to speed up the

process.  The following weighted mask is used:

-1  -1  -1

-1   8  -1

-1  -1  -1

EDGe n         applies an edge enhancement filter to the image using a

3x3 mask.  There are four variations of this filter (n

= 0, 1, 2, 3).  Integer arithmetic is used to speed up

the process.

HIGh n         applies a high pass filter to the image, which has the

effect of removing slowly varying changes in the image,

and leaving behind rapid changes.

HIStogram      applies a histogram equalization filter to the image,

which has the effect of maximizing the contrast across

the whole image.

LOG  n         applies a logarithmic filter to the image which has the

effect of enhancing the contrast in the image.  Useful

on dark images or on images like stars and galaxies.

The parameter that goes with this one is a little

different than the other filters, in that you specify a

factor (default=40) which is multiplied with the log of

the pixel value.  Examples:

MEAn n         applies a mean filter to the image which has the effect

of smoothing out the image.  There are three variations

of this filter (n = 0, 1, 2).  A 3x3 (n=0), 5x5 (n=1)

or 7x7 (n=2) mask is used where the middle pixel is

replaced by the average of the other pixels in the

mask.  Integer arithmetic is used to speed up the

process.

MEDian         applies a median filter to the image which has the

effect of smoothing out the image.  A 3x3 mask is used

where the middle pixel is replaced by the middle of the

nine pixels after they have been sorted.  The sort used

is a simple bubble sort which is terminated halfway

through the sort process since the median value will be

found by then.

POWer n        modifies the pixels of the image by the following

equation:

newDN = numDN * (oldDN/numDN) ^ n

where n is a floating point number.  The default value

is 2.0.  Values less than 1.0 tend to brighten the

image, while values greater than 1.0 tend to darken it.

STAir n        groups pixels together with the net result of smoothing

the image.  The default step increment is 8

SMEar          applies a filter which averages the image in the

vertical direction.  The result is to "smear" the

images down the screen.

UNSharp        applies an unsharp mask to the image, which has the

effect of enhancing edges in the image.  It is the

equivalent of performing a merge with 3 times the

original image minus 2 times the image after it has

been blurred with a 5x5 mean filter.

Examples:

ENH MEAN

ENH EDGE

ENHANCE EDGE 2

ENHANCE BAALKE SCREEN

ENH CON 1

ENH SMEAR SCR

ENH POWER .1   (similar to ENH LOG)

ENH STAIR 2    (creates a binary 2-color image)

4.5.11 ERASE

The command ERASE causes the screen to be erased by setting all of the

pixels to 0, which may not necessarily be black depending on the palette

setting.  If the REFRESH option is selected, then only the refresh buffer

will be erased.  Memory buffers (denoted by letters A through Z) can also

be erased.

Examples:

ERA            erases the screen

ERA REFRESH    erases the refresh buffer

ERA A          erases the memory buffer named A

4.5.12 EXIT

The EXIT command exits IMDISP and returns the user to the MS-DOS

operating system.  The contents of the image display and refresh buffer are

discarded.  The single subcommand NOClear will leave the computer display

in graphics mode, and will not clear the screen.  Same as QUIT.

4.5.13 FILE

FILE takes just a value which is the file name.  The specified file will

be opened, or an error message issued if the file cannot be opened.  The

capability to directly address any sector on the CD-ROM disk by providing

its physical address (minute, second and sector number) is also available

to users of the Microsoft Extensions software.  There is one keyword,

NOMEMORY.  If not given, the FILE command will attempt to put the image

directly into extended memory (where the DISPLAY command will find it).  If

specified, or if the image doesn't fit into extended memory, the DISPLAY

command will read the image from disk.  FILE can now display GIF-format

images, as well.  It does this automatically if the filename extension is

.GIF.

If the FILE command is specified without a filename, or with a filename

mask which contains a wildcard, the program will display a list of file

names in the current directory.  Each file name is preceded by a number.

Typing the number associated with a file name will select that file for

processing.  File names that actually represent lower level directories are

indicated with a <d> symbol after the name.  Directories can be traversed

downward by selecting the number associated with a directory name.  Upward

traversal is by selecting the item identified as PARENT DIR.  This will

move upward in the directory hierarchy.

The prompt mode clears the screen and displays a menu of files (matching

the wildcard mask, if one was used) and commands.  These commands allows

specification of the default drive, a file "mask" to use in selecting file

names for display and options for moving through the list of files when the

current directory contain more than 30 files.  The top line of the display

also indicates how many files are in the current directory.  After exiting

the FILE prompt mode, the previous contents of the display screen can be

recovered by typing "REFRESH".

The FILE prompt mode subcommands:

D    select default disk drive

M    specify a file selection mask to use in displaying files on the

screen.

N    display the next set of file names on the screen if there are

more files in the current directory than can be displayed on one

screen.

P    display the previous set of file names.

Q    quit the file selection mode and return to IMDISP command mode.

The N, P and Q commands are executed immediately.  If the D or M

commands are selected, the user is prompted to enter a drive letter or file

mask.

If the file selected by the "FILE filename" or FILE prompt mode has a

valid label, either PDS or VICAR2, then the number of lines and samples and

the pixel size will be displayed.  If the file is unlabeled then the

program will prompt for the values to use for the number of lines and

samples, the number of bits per pixel, and the number of header bytes.  PDS

images with detached labels can also be displayed.  The specified file

remains the current file until a new FILE command is given.  The FILE

command resets the SET DNLO and SET DNHI values to the minimum and maximum

for the pixel size (normally 0 and 255).  If a specified filename is not

found in the current directory then an error message is displayed.

If the unlabeled image has a regular format, that is a uniform structure

of repeating lines and samples, the user can specify this information when

prompted by the program.  For example, an image named ASTERIA.DAT with one

1422 byte header block followed by an image composed of 370 lines by 450

samples of 8 bit pixels would be processed as follows:

At the COMMAND: prompt type "FILE ASTERIA.DAT".  The program will

display the message:

Input file does not have a proper label.

Input number of lines: 370           enter the value 370

Input number of samples: 450         enter the value 450

Input size of each sample: 8         enter the value 8 (bits)

Input size of header label: 1422     enter 1422 (bytes) for the size of

the foreign label block.

If the size of each sample is greater than 8 bits, the program will ask

whether to swap the byte ordering or not.  16-bit integers written on Unix

or MS-DOS machines generally do not need to be swapped, while 16-bit

integers written on VAX/VMS machines generally do.

The physical address of a data block on the CD-ROM can also be specified

using the syntax 'FILE "CD:mm:ss:bb"', where mm is the minute, ss the

second, and bb the sector block number of the beginning of the desired data

area.  The CD:mm:ss:nn must be in double quotes and the CD must be

capitalized!  This mechanism can be used to access files on non-High Sierra

format disks.  This will only work on systems using the Microsoft

Extensions software.

4.5.14 HELP

HELP displays the online help message.  If followed by the name of a

valid command the message for that command is displayed.  HELP is not

provided for MS-DOS commands (DIR, CHDIR, CD and TYPE).

4.5.15 HISTOGRAM

The HISTOGRAM command produces a plot showing the number of pixel values

in the image at each DN level.  The range of DN values is displayed on the

x-axis and the number of occurrences of a specific DN value on the y-axis

(See Figure 4-1).

The HISTOGRAM command reads through the image, calculates the histogram,

and then displays the plot on the screen.  The histogram is scaled so that

the third highest histogram value is used as the maximum.  It is

recommended that the SUBSAMPLE or other subsetting keywords be used to

speed up histogram calculation, which can be quite slow for large images

and for images stored on CD-ROM.  Histogram plots are displayed on top of

any image currently displayed on the screen, so the user may wish to use

the ERASE command to clear the screen prior to using HISTO.  After the

histogram is displayed, the image can be restored using the REFRESH

command.

27582 |      .

|      ..

|     ....

COUNT  |     .....

|    .......

|    .........

|   ............

|  ...............

--+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

0   64   128   192   256

DN value

Figure 4-1: Sample Image Histogram

By evaluating the histogram, the user can determine the optimum settings

for the SET (DNLO and DNHI) command to produce a useful display.  In the

example above, the image data are clustered between DN values 32 and 144.

Using the default settings of the EGA display mode the colors assigned to

the 2 lowest color values and the 7 highest values (representing DN values

from 0 to 31 and from 144 to 255) would not be used since the image

contains no pixels with these values.

By using the "SET DNLO 32 DNHI 144" command the 16 display levels

available on the EGA display would be assigned to values between 32 and 144

rather than distributed evenly from 0 to 255, and all 16 colors would be

used in the display.

HISTOGRAM takes the following keywords and arguments:

SUBsample n  a integral subsample factor.  For speeding up HISTOGRAM,

use of "SUBSAMPLE 4" is recommended

buffer       displays the histogram of the image in the named memory

buffer

NL n

NS n         number of lines and samples from image

SL n

SS n         starting line and sample in image

CENT         perform the histogram on only the displayed portion of the

image.

Two special keywords can be used with Voyager or Viking images on the

PDS CD-ROM disks.  These will extract histogram values stored with the

image on the CD-ROM disk and reduce the time required for the histogram to

be generated from more than 1 minute to about 3 seconds.  Note that these

commands will not work with Voyager image files in the

\PLANET\JUPITER\ATMOS directory, or with versions of system software

written before April 1987.

VOY   use histogram from Voyager CD-ROM engineering data

VIK   use histogram from Viking CD-ROM engineering data

4.5.16 LABEL

The LABEL command prints the labels for the currently selected image

file on the screen.  It pauses after each screen full of data, and waits

for you to press a key before displaying the next screen of data.  You can

terminate the listing by pressing the letter 'Q' to quit and return to

IMDISP.

4.5.17 LOG

The LOG command controls logging commands to a text file as they're

entered.  The resulting file can be used as a batch file, to repeat the

command sequence, or as a record of what operations have been performed.

The file is automatically flushed and closed when you exit IMDISP.

The LOG command takes the following keywords and arguments:

ON             opens a log file called IMDISP.LOG.  Commands will be

copied into this file until it is either closed or you

exit IMDISP.

FILe  fname    opens a log file called fname (defaults to IMDISP.LOG).

CLOse

OFF            closes the current log file.

4.5.18 MASK

The MASK command is used to mask out pixels on the low or high end of

the color palette.  It is particularly userful when merging images

together.  It sets all the pixels with a DN value above or below a

specified value to a default.

The MASK command takes the following keywords and arguments:

LO n           sets all pixels with a DN value of n or less to the

default DN value of 0.

HI n           sets all pixels with a DN value of n or greater to the

default DN value of 255.

WITH n         overrides the default values for LO and HI, and sets the

pixels to the DN value n.

Examples:

MASK LO 20             Sets the DN values for all pixels with DN values of

20 or less to 0.

MASK LO 20 WITH 255    Sets the DN values for all pixels with DN values of

20 or less to 255.

MASK HI 200            Sets the DN values for all pixels with DN values of

200 or more to 255.

MASK HI 200 WITH 0     Sets the DN values for all pixels with DN values of

200 or more to 0.

4.5.19 MENU

The MENU command reads a file containing a list of file names and places

the user in the MENU mode, where files from this list can be selected for

display.  The MENU mode is similar to the FILE mode, except that the list

of files presented is that provided in the menu file, rather than from a

selected directory.

The MENU mode can be invoked by typing "MENU filename.MNU" at the

COMMAND: prompt.  It can also be invoked on program startup by typing

"IMDISP filename.MNU" to initiate the program.  The menu file must use the

".MNU" extension to startup IMDISP in MENU mode.

Subcommands of the MENU mode are as follows:

#         - Enter the number corresponding to a desired

file name to select the file.

P)revious - display the previous screen of file names

N)ext     - display the next screen of file names

Q)uit     - quit or exit from MENU mode

After selecting and displaying an image, the user can return to the

current menu by typing "MENU" at the COMMAND: prompt.

A menu file can take either of two formats.  The preferred format is a

PDS labelled table file containing file names.  A sample file is shown

below:

NJPL1I00PDS100000000      = SFDU_LABEL

FILE_TYPE                 = TABLE

RECORD_FORMAT             = STREAM

OBJECT                    = FILE_NAME

TYPE                     = LITERAL

LENGTH                   = 80

ENDOBJECT

NOTE                      = "Sample MENU file for IMDISP"

END

L:\OCEAN\SCBMEAN.IMG

L:\OCEAN\WCMEAN.IMG

L:\PLDS\TMCH01.DAT

L:\PLDS\DEM.DAT

L:\PLANET\SATURN\ATMOS\C3497355.IMG

<END OF FILE>

A second abbreviated form may also be used, where only the filenames are

specified in the ".MNU" file.  In this case the file would only contain the

filenames shown after the END statement in the example above.

4.5.20 MERGE

The MERGE command will merge buffers, images or the screen together.  A

scaling option can be provided to control the amount of merging.  If the TO

option is not used then the result will be displayed on the screen by

default.  The result can also be written to a buffer.

MERGE buffer

MERGE source1 [scale1] WITH source2 [scale2] [TO buffer] [FILE]

Examples:

MERGE A                   Merge the last displayed image with buffer A and

display the result back on the screen

MERGE A WITH B            Merge buffer A with buffer B and display the

result on the screen.  Since no scaling parameter

are specified IMDISP will default to 50% of A and

50% of B.

MERGE A WITH B TO C       Merge buffer A with buffer B and store the result

into buffer C

MERGE A WITH B -1         Subtract buffer A from buffer B

MERGE A .75               Merge 25% of the last displayed image with 75% of

buffer A, display the result on the screen.

MERGE A .75 with B .8     Merge 75% of buffer B with 80% of buffer B,

display result on the screen

MERGE B WITH SCREEN       Merge buffer B with the screen

MERGE A 3 WITH B -2 TO C FILE    Subtract twice buffer B from 3 times A,

store the result in buffer.

4.5.21 OVERLAY

The OVERLAY command draws a lat/lon overlay grid on the displayed image.

It is intended only for use with the Ocean Data System, West Coast Time

Series CD-ROM in this program version.

The OVERLAY command has the following arguments:

MAXLAT   Latitude of top of image.

MINLAT   Latitude of bottom of image.

MAXLON   Longitude of left side of image.

MINLON   Longitude of right side of image.

LEGend   Draw a color scale with the overlay.

4.5.22 PALETTE

The PALETTE command is used for changing the colors assigned to

displayable color values.  This is done by modifying an internal color

table which assigns intensity values for red, green and blue to a color

value, The EGA provides 4 intensity values for each of 3 colors, so that 64

different colors can be created.  However, the EGA pixels are only 4-bits

each so only 16 color values can be used simultaneously.

The palette can be modified using the EDIT command and saved and loaded

from a disk file.  The edit mode is invoked by typing PALETTE EDIT.  With

the EGA display, a bar containing 16 colors or shades will be displayed on

the bottom of the screen.  A small square will appear in the middle of the

center color box.  This indicates that this color value is currently

selected for editing.  To select another color value for editing, move the

small square to the appropriate color using either the "4" (left arrow) and

"6" (right arrow) keys on the numeric keypad or the mouse.  The "7" (Home)

and "1" (End) keys move to the beginning and end of the palette.  Pressing

the "Ctrl" key with either the "4" or "6" keys will jump the small square

left or right by 8 color bars.

The color of the current color box is changed with the "R", "G", and "B"

keys (for red, green, and blue).  Upper case letters increase the amount of

the primary color, and lower case keys decrease the amount of the color.

The "8" (up arrow) and "2" (down arrow) keys increase and decrease,

respectively, the amount of red, green, and blue simultaneously.  The left

button on the mouse does the same thing.  Other commands in the palette

edit mode are the "S" and "s" commands, which will shift all colors one

value to the right or left, and the "x" command, which will exchange color

values (the color setting for color 0 becomes the setting for color 15,

while 15 replaces 0, the color setting for color 1 becomes the setting for

14, while 14 replaces 1, etc.).  The up and down arrow keys can also be

used to cycle through the colors available in a given display mode one at a

time.  Type the "." key or carriage return to exit the edit mode.

PALETTE takes the following keywords and arguments:

EDIT n         to interactively adjust an n-color palette.  See edit

description (above) for details

PSEUDOCOLOR n  PS = 0 for default EGA palette,

PS = 1 for gray scale palette,

PS = 2 or 3 for pseudo color palettes (16 levels)

PS = 4, ..., 9 for pseudo color palettes (256 levels)

SAVE filename  saves the specified palette on disk

LOAD filename  loads the specified palette from disk, if no filename is

specified, then list of files is displayed.  See FIL

command for more details.

DIS            displays the current palette on the screen

ERA            erases the current palette from the screen

CYC  n         cycles the color palette

CYC = 0  "blinks" the palette

CYC = 1  blinks four times

CYC = 2  rotates the palette for spectacular results

CYC = 3  rotates and feeds in random colors

CYC = 4  randomizes the palette

CYC = 5  double rotates the palette

CYC = 6  random swaps of the palette

Any key will stop the palette cycling.  A 'S' or 's' key

will save the palette which is currently being

displayed, any other key will restore the palette to its

original colors.

CYCLE itself has two keywords.

FASt     sets the delay between changes to .1 second

(the default is .3).  Works best on machines

with fast video, where flicker is minimized.

TIMe n   runs the PAL CYCLE command for n seconds

(useful in batch files).

The default palette file for LOAD or SAVE operations is IMDISP.PAL.  The

format of the default color palette when stored in a palette file is as

shown below:

NJPL1I00PDS000000784     = PDS_SFDU_LABEL

FILE_TYPE                = TABLE

RECORD_TYPE              = STREAM

FILE_RECORDS             = 28

TABLE_ROWS               = 16

ROW_COLUMNS              =  4

COLUMN_NAME              = (COLOR_NUMBER,                                          RED_VALUE,

GREEN_VALUE,

BLUE_VALUE)

COLUMN_TYPE              = (INTEGER,INTEGER,INTEGER,INTEGER)

END

0   0   0   0

1  64   0   0

2 128   0   0

3 192   0   0

4 192  64   0

5 192 128   0

6 192 192   0

7 128 192   0

8   0 192   0

9   0 192 128

10   0 128 128

11   0 128 192

12   0   0 192

13 128   0 192

14 192   0 192

15 192 192 192

The first column of the palette table is the color value number for the

display, and columns 2, 3 and 4 represent the intensity of the red, green

and blue (respectively) primary colors on a scale of 0 to 255.  With the

EGA display these values are: 0 = off; 64 = low; 128 = medium; 192 = high.

Examples:

PAL PS 1          (Gray scale palette)

PAL PS 7          (Built in color palette #7)

PAL DIS

PAL EDI

PAL CYC 2

PAL CYC 4

4.5.23 PERSPECTIVE

PERSPECTIVE displays the current image as a pseudo-perspective plot.

This is useful for inspecting the topography of an image - brighter areas

will seem to be raised higher than dark areas.

The PERSPECTIVE command has these arguments:

SL  n   displays the plot starting at line n of the image

SCR     reads the image from the screen, rather than from disk (slightly

faster).

4.5.24 PROFILE

PROFILE plots the DN values of pixels located along a line between two

points in the image, i.e., it creates a graph of DN value versus pixel

along the line joining the two points, CURsor mode is used to select the

two endpoints; typing "." or carriage return, or tapping the left mouse

button, selects the current cursor position as the endpoint.  After the

endpoints are selected a line is drawn between the points and the graph is

plotted at the bottom of the screen, showing the DN values on the x-axis

and the positions along the line on the y-axis.

4.5.25 QUIT

The QUIT command exits IMDISP and returns the user to the MS-DOS

operating system.  The contents of the image display and refresh buffer are

discarded.  The single subcommand NOClear will leave the computer display

in graphics mode, and will not clear the screen.

4.5.26 REFRESH

REFRESH redisplays the image plane from the refresh buffer.  When images

are written to the screen they are also written to the refresh buffer.  The

REFRESH command writes this buffer back to the screen, thereby erasing any

graphics or text overlaying the image on the screen.  The refresh buffer

contains as many lines from the display screen as there is room for in

memory.  REFRESH takes no parameters.  It can also be used after the FILE

prompt mode to redisplay the contents of the refresh buffer.  By default,

the refresh buffer is allocated from extended (XMS) memory.  If extended

memory is present, the refresh buffer is on by default.  If there isn't any

extended memory, or not enough extended memory is available, the refresh

buffer is off by default, by may be turned on to use a virtual file.  The

location of the virtual file is given by the environment variable

IMREFRESH.

REFRESH is commonly used after the HELP command, or after a HISTOGRAM or

PROFILE command to remove text and plots from the display screen.

The refresh buffer can be disabled/enabled with the SET command:

SET REFRESH ON

SET REFRESH OFF

4.5.27 ROTATE

The ROTATE command rotates the image through a specified angle.  The

entire image or just the displayed portion can be rotated.  The resulting

image can be clipped to the dimensions of the screen, if desired, and the

borders of the image filled.

ROTATE takes the following arguments and keywords:

n          the angle to rotate the image through.  Must be between 0 and

360 degrees.

SCREEN     rotate the entire screen

CLIP       trim off the parts of the rotated image outside the display

NOFILL     don't blank out the borders of the image

4.5.28 SAVE

The SAVE command copies the contents of the display screen to a file.

On EGA systems the display buffer contains only 4-bits per pixel, so

specifying BP = 8 will write the 4-bit pixels in full bytes, but does not

save full 8-bit pixels even if the input image is an 8-bit image.  Use the

COPIM utility program to produce a full resolution subset of an 8-bit image

in that case.  An 8-bit display device can save an image with full 8-bit

pixels.

SAVE takes the following arguments and keywords, where the filename must

immediately follow the SAVE command:

filename    filename of image to save - if the extension on the

filename is .GIF, the file will be saved in GIF format

instead of PDS format.

FORMAT xxx  saves the file in the specified format, where "xxx" is GIF,

RAW or PDS.

NL n

NS n        number of lines and samples to save

SL n

SS n        starting line and sample to save

BP n        number of bits per pixel in output image

REFRESH     save the displayed image into the refresh buffer

Example: Save a 200 line by 200 sample area from the inner region of an

image in the file SMALL.IMG.  First use the FILE command to select and

display an input image.  Now type:

"SAVE SMALL.IMG SL 100 SS 210 NL 200 NS 200"

to save the portion of the image beginning at line 100 and sample 210 in

the new file SMALL.IMG.

4.5.29 SET

SET is used set various display options.  It may be used to display and

change the DN (pixel value) range that is used to compress the pixel values

to the range appropriate for the display device.  The default, which is

reset for each new file, is the full range of DN values for the particular

pixel format (e.g.,  0 to 255 for byte).

The results of a SET DNLOW or SET DNHIGH command will not take effect

until the next DISPLAY command is performed.

SET can also be used to specify a file name to be used by the browse

command, change the default display resolution or change the number of

samples used to display the current image.

Set takes the following keywords:

DNLOW n

DNHIGH n  where values less than DNLOW are set to color value 0, values

greater than DNHIGH are set to the maximum color value

available, and the color values between are assigned equally

to the DN values between DNLOW and DNHIGH.  You can also use

LOW and HIGH instead of DNLOW and DNHIGH.

BROwse filename

sets a file name to be used for the browse file.  This file

name will override the default file name, which is

'c:browse.cmd'.

NS n      sets the number of samples to be used for the next DISPLAY

command.

REFresh ON

REFresh OFF

toggles the state of the refresh buffer on or off.  If IMDISP

detects the presence of extended memory (XMS - managed by a

device driver like HIMEM.SYS or QEMM), the refresh buffer

will default to ON.  If no XMS is present, the default is

off, and turning the refresh buffer on will cause IMDISP to

set up a scratch file to be used as the refresh buffer.

DELay ON

DELay OFF (default)

enables/disables the time delay before executing commands.

Most useful when used in batch file, because it allows the

commands to play slowly instead of as fast as possible.

PROmpt ON  (default)

PROmpt OFF

enables/disables the display of the command names before

execution.  Most useful when used in batch files, because it

prevents the command prompts from appearing on the screen.

DISPLAY name

sets the current display device.  You might use this to

change from Super VGA resolution to VGA if you wish to have

bigger pixels for displaying images with the BROWSE command,

for example.  To do this, you would type the command SET DIS

VGA.  The change will take place immediately.  The current

values for DISPLAY are

CGA         (640x200x2)

EGA         (640x350x16)

EGA480      (640x480x16)

VGA320      (320x200x256)

VGA         (640x480x16)

ATI640      (640x480x256) - ATI VGA Wonder Board (512K)

ATI800      (800x600x256) - ATI VGA Wonder Board (512K)

ATI1024     (1024x768x16) - ATI VGA Wonder Board (512K)

EVGA512     (512x480x256) - Everex EV-673 Board (256K)

EVGA640     (640x400x256) - Everex EV-673 Board (256K)

ORCHID      (640x480x256) - Orchid ProDesigner+ Board (512K)

ORCHID800   (800x600x256) - Orchid ProDesigner+ Board (512K)

ORCHID1024 (1024x768x256) - Orchid ProDesigner+ Board (1MB)

PARADISE    (640x480x256) - Paradise SuperVGA Board (512K)

(640x400x256) - Paradise SuperVGA Board (256K)

PGA         (640x480x256) - PGA Board

TRIDENT     (640x480x256) - Trident-based SVGA boards (512K)

VESA       (1024x768x256) - VESA SVGA Interface (1MB)

PALette directory

Sets the default palette directory.  This is used with the

PAL LOAD command.  If PAL LOAD is used with no parameters,

IMDISP will go to the default palette directory and display

the files there.

BUFfers directory

Sets the directory to hold the buffers as files instead of

memory buffers.  Useful when not enough extended (XMS) memory

exists to create the buffers.

For example, using the default color palette and an EGA display with 16

color values available, the command "SET DNLO 100 DNHI 164" will assign DN

values below 100 to black and DN values above 164 to white, and spread the

16 color values between the DN limits, thus each color value will represent

4 DNs (value 0 = DN 100 to 103; value 1 - DN 104 to 107, etc.).

If values other than the default are used then the images will display

more slowly because scaling divisions are required.  SET with no parameters

will display the current values.

The DNLO and DNHI values are reset to the minimum and maximum for a

given pixel size when a FILE command is issued.

4.5.30 SLANT

SLANT displays the image slanted either to the left or right.  This is

done by shifting each line of the image by 1 pixel.  It is particularly

useful for finding errors in image headers.

Examples:

SLANT                (slants image to the right)

SLANT LEFT

4.5.31 STRETCH

STRETCH sets the palette to a gray scale stretched between the two

specified pixel values.  Stretch is most useful on the PGA, VGA and

SuperVGA displays where 16 or more gray levels are available.  On the EGA

display the stretch command activates the gray level palette which provides

only 4 gray levels.  Use the PAL PS 0 command to return the display to the

default palette after performing a stretch.  Note that STRETCH only affects

the color look-up tables in the display hardware, rather than modifying the

pixel values as it reads the data, so it is very fast.

STRETCH takes the following keywords :

LOW n

HIGh n   the low and high DN values of the stretch.

These pixel values refer to the DN values in the current image display,

not necessarily in the original image (scaling may have been involved).

For example with the EGA display the DN values in the display range from 0

to 15, even if a byte image is being displayed.

The STRETCH command is not recommended for use on EGA systems.

4.5.32 SYSTEM

The SYSTEM command is used to send a command to MS-DOS.

For example "SYS L:" will set the L: drive as the current default drive.

If there is not enough memory available for the system command to be

executed an error message is displayed.  The SYS command will not work on

floppy disk systems unless the file "COMMAND.COM" is present on the floppy

disk with the IMDISP program.

The SYSTEM command in this version of IMDISP will attempt to free as

much of memory as possible, by swapping most of the program to scratch

space.  It tries to find Extended memory (XMS), Expanded memory (EMS) or

disk space, in that order.  If it succeeds, only a small amount of IMDISP

remains in memory, leaving far more memory for DOS functions that previous

versions.  If the attempt to swap fails, the program will try to swap to

DOS with the available memory.  Currently, you must have about the same

amount of free XMS, EMS or disk space as you have memory free before

running IMDISP (i.e., IMDISP allocates all available memory to itself when

it starts, and requires that much scratch space when swapping out).

The swap process, if it attempts to write out to a scratch disk file,

will use the directory specified by the environment variable IMBROWSE.  If

IMBROWSE doesn't exist, IMDISP will write the scratch file to the root

directory of drive C:.  The scratch file is deleted when you return to

IMDISP (using the EXIT command at the DOS prompt).

4.5.33 TEXT

The text command can be used to display a string of text on the screen.

The text string can be entered on the command line, enclosed in apostrophes

or quotes (e.g., TEXT `This is a text string').  If the text string is not

entered on the command line the program will prompt for the string to be

displayed.

Once the command is issued and the text string entered the program is

placed in CURSOR mode so that the display position can be selected.  This

is done by using the arrow keys to move the cursor to the desired location.

After the position is selected, the user types a period (.) or carriage

return and the text will be written on the display screen.

The TEXT command has one argument and two optional keywords.

LINe      The line number to position the text at.

SAMple    The sample number to position the text at.

NOPrompt  Will display text without the command line prompt.

'text...' The text string to be printed, enclosed in quotes or

apostrophes.

The text command writes over image data in the display and refresh

buffer, thus the underlying portion of the image is lost.

4.5.34 TYPE

The TYPE command is used to type the contents of a text file on the

display screen, using the standard MS-DOS type command.

Its argument is passed to MS-DOS for execution.  At the end of the type

operation the screen will display "Type carriage return to continue:" which

will return to the IMDISP command mode.  If there is not enough memory

available for the system command to be executed an error message is

displayed.

4.5.35 NEG

The NEG command is used to invert the current color palette, turning the

image into a negative.  It takes no keywords or arguments.

4.5.36 PLOT

The PLOT command is used to draw plots of image lines and spectra on the

screen.  If the current file is an image, you must specify which line you

wish to plot.

PLOT takes the following keywords:

LIN n      plot line n of the image (required for 2-D files)

SYMbol x   draw the plot using character x instead of connecting the

points with a line

ZOOm       prompt user to zoom in on a region of the plot after it is

drawn

OVErlay    overplot - do not erase the current plot

MAX n      set the maximum y value in the plot

MIN n      set the minimum y value in the plot

COLor n    use DN value n to plot the data

SPEctrum   analyze the plotted spectrum by doing baseline removal and

integrating line flux.

Example:  to compare lines 200 and 210 of the current image on the same

vertical scale, first plot line 200 (as a line) with the command

PLOT LINE 200 MIN 0 MAX 255

Now overlay the plot of line 210 using "+" signs and color 10 of the

current palette with the command

PLOT LINE 210 OVERLAY MIN 0 MAX 255 COLOR 10 SYMBOL +

4.5.37  WINDOW

The WINDOW command lets the user define a subimage with the cursor keys

or a mouse.  The mouse button (or enter key) sets the starting point and

then the lower corner is "dragged" to the desired position.  The window

stays in effect until turned off with the CANCEL subcommand.

WINDOW takes the following subcommands:

CANCEL      cancels the defined window and sets commands back to the

full screen

S1 n

S2 n        defines the starting and ending samples of the window

L1 n

L2 n        defines the starting and ending lines of the window

4.6 OTHER UTILITY PROGRAMS

There are several utility programs also provided on the distribution

disk.  These programs can read PDS labeled, VICAR2 labeled, or unlabeled

images.  The output images are always written with PDS ODL labels.  The

programs prompt for the necessary input values.

4.6.1 COPIM

COPIM is a simple image copying program.  It can copy the whole image,

for example to put PDS labels on a foreign image), or extract a window from

the image.  It can subsample the image with an integral subsampling factor.

The program prompts for the input and output file names, the window to

extract (just type return for the whole image), and the subsampling factor

(the default is 1).  If the input file is not in VICAR2 or PDS image format

the program will also prompt for the number of lines and samples, pixel

size and header size of the image.

For example, to extract the first 100 lines and samples from the file

LOGO.IMG the following commands would be used:

COPIM

Input Image: LOGO.IMG

Lines: 350 Samples: 340 Bits per pixel: 1

Output Image: LOGOSUB.IMG

Starting line and sample, number of lines and samples:

1 1 100 100

Line Subsampling factor (1): 1

Sample Subsampling factor(1): 1

4.6.2 CONVERT

CONVERT is simple pixel format conversion program.  It converts an image

to an integer (16 bits), byte (8 bits), nibble (4 bits), or binary (1 bit)

image.  It can also perform a user specified scaling.  The program prompts

for the input and output file names, the output format (must be 1,4,8, or

16 bits), and the input and output numerical ranges for scaling.  The

default scaling values are appropriate for the data types being used.

For example, to convert MONTAGE.IMG to an 8-bit per pixel image and

scale the output pixels evenly between the values of 0 and 127 the

following commands would be used:

CONVERT

Input Image: MONTAGE.IMG

Lines : 350    Samples : 640 Bits per pixel : 4

Output Image: MONTBIG.IMG

Output bits per pixel (1, 4, 8, 16) : 8

Input DN range (O 15) :

Output DN range (O 255) : 0 127

Chapter 5

CREDITS

IMDISP is maintained and updated (since version 4.4) by a small group of

volunteers -- Mike Martin, Ron Baalke (JPL) and Archie Warnock (Hughes STX

Corp).  It was originally written by Mike Martin, Frank Evans and Dan

Nakamura of the Jet Propulsion Lab.  The plotting code was originally

written by Ed Esfandari (Interferometrics).  Paradise and Trident video

drivers were added by Gregg Gunnells of the USGS.  Jerry McFaul (USGS), Ed

Grayzeck (U of MD) and Nick Beser (JHU/APL) have been valiant testers of

the program through its many iterations, and have made many suggestions for

improvements along the way.

IMDISP would not have its current capabilities if it weren't for the

generous efforts of several software authors who have seen fit to donate

their work into the public domain.  Through their work, we are able to keep

IMDISP in the public domain, free to anyone who wants it.  We want to

publicly thank all of them here:

Gershon Elber     - GIF I/O routines (GIF_LIB.ZIP)

Michael Walraven  - High resolution timer (TIMERHI.ZIP)

Max Medley        - Mouse interface code

James Birdsall    - EMS, XMS allocation code (EMSLB215.ZIP, XMSLB121.ZIP)

Marty Del Vecchio - EMS/XMS swap routines (SWAP300.ZIP)

And, of course, we would also like to thank our users, from whom we

continue to receive ideas, bug reports and fixes.  They exercise the

program in ways we never imagined.

GIF and 'Graphics Interchange Format' are trademarks of CompuServe,

Incorporated, an H&R Block Company.

Appendix

COMMAND SUMMARY

Within this command summary listing, brackets ([]) indicate optional

parameters or keywords.  Filename is the name of a file in the current

directory or a fully qualified filename which can include drive letter and

pathname.  Directory name is an MS-DOS directory path specification, a

valid directory mask a specification containing wild card characters (*, ?)

per MS-DOS rules for wildcard use in the DIRECTORY command.  The symbol "n"

is used to indicate an integer value.

BATCH        [filename]

BRIGHTEN     [n]

BROWSE       [directory mask] [SIZe n] [SUB n] [NOLabel]  [DNLOw n]

[DNHIgh n] [PAUse n] [ALL] [SELect] [FILe] [BUFFERS]

[AUToset n]

CD           [directory name] [..]

CHDIR        [directory name] [..]

COPY         [buffer [TO buffer]] [FILE]

CURSOR       [DSL n] [DSS n]

DARKEN       [n]

DIRECTORY    [directory mask] [/w]

DISPLAY      [SUB n] [SL n] [SS n] [NL n] [NS n] [BP n] [CENTer] [DSL n]

[DSS n] [ZOOm] [FLIP] [NOPrompt] [NOMemory] [AUToset]

[SOUrce] [CURsor] [buffer]

ENHANCE      [BAAlke] [CONvolution n] [EDGe n] [HIGh n] [HIStogram]

[LOG n] [MEAn n] [MEDian] [POWer n] [STAir n] [SMEar]

[SCReen] [UNSharp]

ERASE        [REFresh] [buffer]

EXIT         [NOClear]

FILE         [filename[.GIF]] ["CD:mm:ss:bb"] [NOMemory]

HELP         [command name]

HISTOGRAM    [SUB n] [SL n] [SS n] [NL n] [NS n] [BP n] [CENter]

[VOYager] [VIKing] [buffer]

LABEL

LOG          [ON] [FILe filename] [CLOse] [OFF]

MASK         [LO n [WITH n]] [HI n [WITH n]]

MENU         [filename]

MERGE        [buffer [scale] [WITH buffer [scale] [TO buffer]]] [FILE]

NEG

OVERLAY      [MAXLAT n] [MAXLON n] [MINLAT n] [MINLON n] [LEGend]

PALETTE      [EDIT n] [PS n] [LOAd filename] [SAVe filename] [DISplay]

[ERAse] [CYC n [FAST] [TIME n]]

PERSPECTIVE  [SL n] [SCR]

PLOT         [LINe n] [SYMbol x] [COLor n] [MAX n] [MIN n] [OVErlay]

[ZOOm] [SPECtrum]

PROFILE

QUIT         [NOClear]

REFRESH

ROTATE       [n] [SCReen] [NOFill] [CLIp]

SAVE         filename[.GIF] [FORmat fmt] [SL n] [SS n] [NL n] [NS n] [BP n]

[REFRESH]

SET          [DNLO n] [DNHI n] [BROwse filename] [DIS name] [NS n]

[PAL dir] [REFresh ON/OFF] [DELay ON/OFF] [PROmpt ON/OFF]

[BUFFERS dir]

SLANT        [RIGht] [LEFt]

STRETCH      [LO n] [HI n]

SYSTEM       [MS-DOS command]

TEXT         [LINe n] [SAMple n] "text" or 'text' [NOPrompt]

TYPE         filename

WINDOW       [CANcel] [S1 n] [L1 n] [S2 n] [L2 n]

MS-DOS ENVIRONMENT VARIABLE SUMMARY

All of these environment variables control options for IMDISP.

SET IMDISP=<display-type>

SET IMBROWSE=<subdirectory>

SET IMPALETTE=<subdirectory>

SET IMREFRESH=<subdirectory>

SET IMHELP=<subdirectory and/or filename>