IBM CD-ROM DEVICE TEST TOOL

THE MENU

==========

FILE MENU:

---------

PREFERENCES:    Here you can set the default numbering system of numeric

entry fields, as well as most numeric output, to either

decimal or hexadecimal format.  This is merely for input

convenience and may be overridden at any time.  All

HEX input should be preceeded by '0x' and all decimal

input should be entered as is.  The same is not true of

the addressing mode.  The appropriate format must be used

during input, and may not be overridden.  This will be

obvious.  Output defaults to your selection.

SAVE TEXT:      Allows screen output to be saved to a file.  After saving,

the user is given the option of erasing the text window.

Once the window is cleared, that text is lost.

CLEAR TEXT:     Clears the text in the window.  Once cleared, the text

cannot be retrieved.

DATA MENU:

---------

DOS READ SECTORS:

This will prompt the user for a starting sector from

which to begin reading the currently opened CD-ROM, and

a count of 2048-byte sectors to read.  The DosRead function

is used to read the disk as a volume.  The returned data

will be displayed in the text window.

DOS READ FILES: The user is prompted for a filename, an offset from the

beginning of the file, and a byte count to be read from

the file.  The DosRead function is used to read the file.

The returned data will be displayed in the text window.

READ LONG:      The user is prompted for a starting sector and a count of

2352-byte sectors to read from the currently opened CD-ROM.

The ReadLong IOCtl [Category 8, Function 72] is used to read

the data, and it is displayed in the text window.

READ LONG PREFETCH:

Same as READ LONG except that the data is not returned to

a buffer, as in ReadLong, therefore no data is displayed

on the screen.  Uses IOCtl [Category 8, Function 73].

GET SECTOR SIZE:

Queries the sector size from the currently opened CD-ROM

using IOCtl [Category 8, Function 63].  The returned value

is displayed.

GET UPC CODE:   Queries the UPC code of the CD in the currently opened

CD-ROM drive using IOCtl [Category 8, Function 79].  Not all

CD's have this so the returned data may not truly be a UPC

code.

GET VOLUME SIZE:

Queries the volume size from the currently opened CD-ROM

using IOCtl [Category 8, Function 78].  The returned value

is displayed.

READ HEAD LOCATION:

Finds the location of the drive head of the currently

opened CD-ROM and displays it to the text window.  Uses

IOCtl [Category 8, Function 70].

SEEK:           Prompts the user for a sector on the CD in the currently

opened CD-ROM.  Uses IOCtl [Category 8, Function 50].

AUDIO MENU:

==========

AUDIO CHANNEL CONTROL:

Allows user to configure the audio channel assignments and

volume levels of the currently opened CD-ROM device.  Uses

IOCtl [Category 81, Function 40].

AUDIO CHANNEL INFO:

Queries the currently opened CD-ROM device for the channel

mapping and volume parameters and displays them in the

text window.  Uses IOCtl [Category 81, Function 60].

AUDIO DISK INFO:

Queries the currently opened CD-ROM device for the high

and low track numbers, along with the lead-out track

address, and displays them in to text window.  Uses

IOCtl [Category 81, Function 61].

AUDIO STATUS INFO:

Queries the addresses of the last play/continue command

executed by the current CD-ROM device, along with the

audio status word, and displays them in the text window.

Uses IOCtl [Category 81, Function 65].

AUDIO-SUBCHANNEL INFO:

Queries and returns audio-subchannel information about the

current CD-ROM device and displays it to the text window.

Uses IOCtl [Category 81, Function 63].

AUDIO TRACK INFO (SINGLE):

Prompts the user for a track number, then queries the

current CD-ROM device for information about that track.

The info is displayed in the text window.  Uses IOCtl

[Category 81, Function 62].

AUDIO TRACK INFO (ALL):

Same as above except that the user is not prompted for a

track number, and data on all available tracks is displayed.

PLAY AUDIO:     The user is prompted for a starting and ending sector,

defining the play region.  Uses IOCtl [Category 81, Function

50] to start playback on the currently opened CD-ROM device.

RESUME AUDIO:   Uses IOCtl [Category 81, Function 52] to resume playback on

the current CD-ROM device.

STOP AUDIO:     Uses IOCtl [Category 81, Function 51] to stop audio playback

on the currently opened CD-ROM device.

DEVICE MENU:

===========

CHANGE DRIVE:   Prompts the user for the drive letter of the desired CD-ROM

device and attempts to open it using DosOpen.

CHANGE MEDIA:   This simply re-opens the current CD-ROM device.  This should

be done whenever the media is removed and replaced.

IDENTIFY DRIVER:

Uses IOCtl [Category 80, Function 61] to query the driver ID

string and displays it to the text window.

RESET DRIVE:    Uses IOCtl [Category 80, Function 40] to reset the current

CD-ROM device.

GET DEVICE PARAMETERS:

Uses IOCtl [Category 8, Function 63] to retreive either the

current BPB or the recommended BPB, whichever the user

selects, and displays the returned bytes in the text window.

STATUS:         Uses IOCtl [Category 80, Function 60] to retreive the device

status word. The status word, along with a bit-by-bit

breakdown is displayed to the text window.

CLOSE TRAY:     Uses IOCtl [Category 80, Function 45] to close the tray of

the currently opened CD-ROM device.

EJECT:          Uses IOCtl [Category 80, Function 44] to eject the media

from the currently opened CD-ROM device.

LOCK DOOR:      Uses IOCtl [Category 80, Function 46] to lock the door of

the currently opened CD-ROM device.

UNLOCK DOOR:    Uses IOCtl [Category 80, Function 46] to unlock the door

of the currently opened CD-ROM device.

UTILITIES MENU:

==============

ADDRESS CONVERSIONS:

This utility allows conversion between logical block

(sector) addresses and redbook (Minute:Second:Frame)

addresses.  Simply enter a know address in either

format, and click on the arrow pointing to the other

format's entry field(s).  The equivalent address will

appear in that field.

* NOTE: This requires IOCtl [Category 81, Function 62] to be

functioning properly so that the location of the first

readable sector can be determined.  This is true of any

function of the test that needs to convert from one address

format to another.  A message box will appear indicating

failure if one should occur.

DETERMINE CRC FOR A FILE:

This utility allows calculation of CRC values for any

files on any accessible disk.  To determine a single

file's CRC value, simply enter the full path and filename

and leave the box unchecked.  One way to conveniently

find the CRC's for several files is to create a text file

that contains a list of the full paths and filenames of the

files you want, one file per line.  The CRC values will be

appended to the end of each line, and the resulting file

will be saved as 'FILELIST.NEW' in the current directory.

If a '?:' is put in place of 'd:', where d is the drive

letter of the path, the current CD-ROM device's letter will

be used.  EXAMPLE:  "?:\config.sys" becomes "F:\config.sys"

when F: is the current CD-ROM device.  The 'FILELIST.NEW'

file can be used as a list file for the multi-threaded

stress test.

STRESS MENU:

===========

MULTI-THREAD TEST:

This test is intended to verify data integrity when multiple

threads simultaneously access the same CD-ROM device.  Up

to 10 threads may be specified.  If the "Use first n..."

option in the Input Setup section is selected, the test

will find the first n files on the disk, where n is the

number of threads selected to run, and will open them for

use with the test threads.  CRC and pattern checking is

not available when using "first n" because there is no way

of knowing what pattern or CRC to compare to.  For any of

the verification options, a file containing a list of files,

and optional CRC or pattern values, may be used instead of

the "first n" option.  If the "Stop on error" option is

checked, the entire testwill end if a single error is

encountered.  All test setup info and events are recorded in

a logfile specified by the user.  The scratch directory is

needed for copy operations, and should be large enough to

accommodate all of the files you wish to use.  Copies will

be deleted at the end of the test.

PERFORMANCE MENU:

================

PERFORMANCE TEST:

This function determines the data transfer rate and CPU

utilization of the driver for the drive containing the

file specified.  The primer block size determines how much

of the file to read prior to starting the test.  After

priming, the file pointer returns to the beginning of the

file and the test begins.  The normal block size is the

number of bytes to read per DosRead call.

* NOTE: Transfer rate values tend to be a little low.  Recommended

file size is > 2M.

CREATING A FILE LIST:

====================

A file list is intended for use with the multi-threaded test.  It

allows specification of several filenames to be used during the test,

as well as CRC code information or a pattern to match.  One way to

create a list file is to simply make a text file that is nothing more

than a list of full pathnames of the files you want to use.  The first

file will be used by thread 1, the second by thread 2, and so on.  If

more threads are specified than filenames provided, then the number of

threads will be reduced to the number of files specified.  An example

of a simple file list is shown below:

--------------------------------------------------------------------------------

#

#  <--  Lines beginning with '#' are considered comments

#

E:\FILE1.DAT

E:\FILE2.DAT

E:\FILE3.DAT

#       End of file

--------------------------------------------------------------------------------

The above file will allow up to 3 threads to simultaneously run during

the multi-threaded test.  Duplicate filenames may be used to allow

several threads to access the same file.  If you have multiple drives

to test with the same CD, you can put a '?' in place of the drive letter

to indicate the files reside on the currently opened CD-ROM device, as

shown below:

--------------------------------------------------------------------------------

?:\FILE1.DAT

?:\FILE2.DAT

?:\FILE3.DAT

--------------------------------------------------------------------------------

The above style may NOT be used with the Verify from CRC or Verify from

Pattern options.  To use these options, you must know the CRC codes for

each file, or the repeating pattern that makes up each file.  There is

a utility under the UTILITIES menu that will compute the CRC's for you

and place the codes properly in a list file like the examples shown

above (refer to the above section on computing CRC's).  Simply supply

the utility with the name of your simple file list and check the box,

indicating that you want multiple CRC's computed.  Then, rename the

resulting file as you see fit.  Alternatively, you can manually enter

the CRC codes, but they must be in the HEX format shown below:

--------------------------------------------------------------------------------

?:\FILE1.DAT    0xff87

?:\FILE2.DAT    0x993a

--------------------------------------------------------------------------------

A file like the one shown above may now ONLY be used with the Verify

from CRC option or the Verify from Pattern option.  It will not work

with the various copy options, and vice versa, the copy option files

will not work with CRC or Pattern.  Patterns must be entered manually,

and must be in the format shown above.  Only 2-byte patterns are

supported, so single-byte patterns must specify the byte two times.

For example:  A repeating hexidecimal FF would be represented by

0xFFFF.