.*	SCCSID = @(#)vpol.doc	13.11 90/10/11

.* symbol definitions

.im ..\..\equate.sci

.im ..\..\macros.sci

:h3.&vpol. Architecture

:h4.Problem Description/Objectives

DOS applications sometimes poll for input and waste CPU time

in a multitasking system.  &vpol. is designed to determine when

DOS applications are idle and to prevent idle VDMs from hogging

the CPU.

&vpol. can never know for certain when a DOS application is idle unless

the application volunteers to yield. In other cases,

&vpol. must guess when an application is idle.

.cp 5

&vpol. goals&gml.

:ul.COMPACT

:li.Catch applications when they are idle

:li.Avoid considering an application idle when it is doing useful work

:li.Provide better idle detection than single DOS box &os2.

:eul.

.cp 5

:h4.Solutions/Justification

.*

.cp 5

The &vpol. component is found in the &vkd. although it is logically

separate.  Idle detection code is also found in the DOS kernel.

All idle detection code is marked by the phrase "VPL Idle detection"

in comments.

.*

.cp 5

The following section lists the idle detection cases

that must be handled and how they are detected.

:ul.

:li.Application loops performing INT 16H polling

VKBD reports INT 16H poll requests and &vpol. tracks

the sum of weighted reports over time.

When peeks occur at a very high rate, the application

is deemed idle.

A single VDHQuerySysValue clock tick is used for the

test period (31 msecs).  &vpol. checks for a new test

period only when it is called to report an idle event.

It does not require any periodic timer notification

to control test intervals.

VKBD reports a peeks by passing a weight to &vpol.  The weight is

determined by doing a callibration that determines how may peeks

can be done in a test period.

Passing a weight in peek reports

allows other devices

to also callibrate and use a weight to report peek events.

.cp 5

:li.DOS in console read loop

DOS reports when in the console read loop by making an

SVC call.

.cp 5

:li.Application in loop doing INT 28H spool calls

DOS reports when repeated INT 28H's originate from outside

DOS.  Only INT 28H from outside DOS indicates idle

activity since DOS also does INT 28H during console

output.

.cp 5

:li.Application does INT 2FH, AX=1680H yield

DOS reports INT 2FH using an SVC call.	(Note: This code

should be moved to the &vdmmgr.).

.cp 5

:li.Application in INT 21H, service 0bh console peek loop

DOS increments a variable when this INT 21H service is called.

&vpol. uses this count to determine when a high rate

of these calls is being done.  This service causes too

many ring transitions to catch through INT 16H peek counting.

:eul.

When an application is judged idle.  It can either be made to

sleep for a time or it can have its priority reduced to idle class

for a time.  In either case, interrupts are simulated

to the VDM even while it is judged as idle.

Some VDD's report "busy" activities to &vpol..

When busy events are reported,

a VDM judged as idle is immediately restored to the non-idle state.

For example, if a keystroke occurs in a VDM, that VDM is no

longer considered idle.

Busy reports also prevent VDM's from being considered

idle for some time after they occur.  Examples of

busy activities reported are non-peek DOS service calls,

keyboard input, printer output, COM IO, and mouse button pushes.

If a VDM is considered idle for a prolonged period of time,

the amount of time it sleeps or runs at idle class

is increased gradually to give it less CPU time.

.*

.*:h4.Architectural Review

:h3.&vpol. Design

:h4.Design Overview

.*

Idle detection handles 2 cases&gml.

:ul.COMPACT

:li.Calls requesting a timeout.

:li.Polling events that occur at a high rate.

:eul.

.*

:h5.Calls requesting a timeout

:ul.COMPACT

:li.INT 2fh AX=1680h idle call from application

:li.DOS call when in its console read loop

:eul.

In both of these cases,

&vpol. notes the reason the VDM is considered idle and

either forces it to sleep for a time or reduces

its priority to idle class for a time.

Busy reports wake the VDM immediately.

:h5.Polling events that occur at a high rate

:ul.

.cp 5

:li.Peek loop detected by VDHReportPeek calls

:ul.COMPACT

:li.VDHReportPeek notification count exceeds a threshhold

:li.No application "busy" activity for some time

:eul.

.*

&vdds. notify &vpol. when polling occurs by calling VDHReportPeek

and passing a weight for the event.

&vpol. keeps a sum

of the weighted reports within a single VDHQuerySysValue timer tick.

When a peek report is made, &vpol. first checks to see if

a new test period has begun and resets counters if it has.

If the sum of reported values exceeds a threshhold in

a test period, the VDM is considered to be idle

and is made to sleep until

a busy report occurs or a timeout expires.

.cp 5

:li.INT 28h's

:ul.COMPACT

:li.More than 16 (non DOS) INT 28h's in test period

:li.No application "busy" activity for some time

:eul.

&vpol. waits for the second

notification in a test period to make sure the VDM

is looping.

It then changes the VDM's priority class to idle

and arranges for a timeout.

If the timeout occurs, the use of idle priority class is ended.

VDHSetPriority arranges to restore the proper priority and

ensures that other VDD's can temporarily raise priority

to do activities like simulating interrupts.

.cp 5

:li.INT 21h service 0bh

:ul.COMPACT

:li.High rate of service 0bh calls in period

:li.No recent busy activity

:eul.

This service is slower than INT 16h due to numerous

traps into ring 0 and so requires special handling.

When &vpol. resets its counters at the start of a test

period, it checks the variable incremented

by DOS when service 0bh is called.  If this counter

is high, the peek counter is reset to a high value

for the next test period so that INT 16H peek detection will

catch the VDM if it continues to poll.	Once the

application has been caught, the VDHReportPeek probation mechanism

keeps the start peek count high so that service 0bh

continues to be caught through INT 16H peek reports.

:eul.

Idle VDMs sleep in VDHWaitVIRR's.  If they awake

to deliver hardware interrupts, they are put back

to sleep in a return hook after interrupt simulation.

.cp 5

INTERFACES

The principle interfaces to &vpol. are&gml.

:ul.

.cp 5

:li.VDHReportPeeks

Receives reports of polling events.  When a threshhold

is exceeded in a test period, the VDM is considered

idle.

.cp 5

:li.VDHWakeIdle

Notes that the application is busy.  The VDM is awakened

if sleeping for being idle and has its priority restored

if priority had been dropped.

.cp 5

:li.VPLDosWaitIdle

Accepts reports of INT 28H's and INT 2fh yield(AX=1680h).

.cp 5

:li.VPLDemInit

Accepts address for DOS "not busy" variable and

coordinates INT 16h callibration with DOSKRNL.

.cp 5

:li.VPLNoIdle

Turns off idle detection for a period of time.

.cp 5

:li.VPLDosReqProc

Returns the entry points for VPLDemInit, VPLDosWaitIdle,

VDHWakeIdle, and VPLNoIdle.  This allows calls from the

kernel and from VDD's loaded before VKBD.

:eul.

.cp 5

ROLE OF DOSKRNL

DOSKRNL aids in idle detection in a number of ways.

All of these activities could easily be moved out of DOS

to reduce the amount of special DOS code needed for &os2..

.cp 5

DOSKRNL provides the following&gml.

:ol.

:li.Calls &vpol. from DOS console read loop.

Another component could provide this service

by prehooking INT 21h to see when DOS looping

may be about to begin.

.cp 5

:li.Clears "not busy" variable when a

non-peek service request is made by an application.

This variable is

reset at the start of each test period

by &vpol..  Another component could provide this

service by prehooking INT 21h.

.cp 5

:li.Increments the "not busy" variable when service 0bh is requested.

This variable is the same one DOSKRNL clears when it receives any

non-polling request.  Another component could provide

this service by prehooking INT 21h.

.cp 5

:li.Reports repeated INT 28h calls from outside DOS.

Another component could provide

this service by prehooking INT 28h, but the service provider

would need to be able to distinguish DOS's CS from application's

CS.

.cp 5

:li.Reports INT 2fh, AX=1680h.

This function should be shifted to MVDM

which already handles other INT 2fh calls.

.cp 5

:li.INT 16H peek callibration.

At the end of DOSKRNL creation, DOS calls &vpol..

DOS supplies the V86 address of the "not busy" variable

discussed above.  A return value indicates the number

of INT 16H peeks DOSKRNL should perform.  &vpol.

is then called again after the peek loop and the process is repeated

until &vpol. requests 0 peeks.	This function could

be moved to a V86 mode stub in VKBD.  The stub

would need to be able to make an SVC call to

contact &vpol. to find out how many INT 16h's to do.

:eol.

.cp 5

PROBATION

When an application is judged idle, it still must be allowed

to run periodically to let it check for input and to let INT 28h

handlers run.  Some input, like keystrokes cause the VDM to wake

immediately because they are reported to idle detection, but other

input is not reported.

When judged idle, the VDM will be placed on one of three kinds

of probation.

:ul.COMPACT

:li.DOS console read probation

:li.INT 28h/ INT 2fh yield probation

:li.VDHReportPeek probation

:eul

The longer a VDM has been on probation for being idle, the longer

it will sleep or run at idle priority each time it is judged idle.

A VDM is removed from probation when "busy" activity is reported,

or when some period of time has passed since it has been judged

idle.  It also restarts probation when the type of probation

changes.

For VDHReportPeek probation, the counter for peek events

is reinitialized to a value near the threshhold while on probation.

This ensures the VDM will be caught more quickly when in a loop.

Periodically, while on VDHReportPeek probation, the counter will

be reset to 0 giving the application a normal test period.

.cp 5

CALLIBRATION

Peek events are weighted so that when an application peeks

in a tight loop, the sum of the reported weights

exceeds 64K in an infoseg clock tick.  To determine the weight

to use, VDD's that report must callibrate based on actual calls

in a VDM.  This rate is highly dependent on the number of

traps to ring 0.  This means the rates on different machines

in real mode may have no relation to the rates in VDMs.

&vpol. arranges to callibrate INT 16H peeks for VKBD.

Ideally, callibration should be moved to a tool that

would run at installation time.  This would require that

VDD's can retrieve the value saved by the tool running in

a VDM.

The threshhold used to judge an application as idle

is a % of 64K.	The user is able to set this level

with an advanced property.  A value of 75 means that

the VDM is considered idle when it performs polling

at 75% of the maximum possible rate.

Setting the value to 100% turns idle detection

off.  A second advanced property allows users to set

a delay for idle detection.  Idle detection will turn

off for this number of seconds every time busy activity

is detected.

.cp 5

VIDEO PEEK TIME PROCESSING

The &vvd. uses INT 16H peek time to do optional checking

for screen changes in windowed VDMs.

This slows down the peek rate an application exhibits,

sometimes so much so that idle applications do not

exceed the threshhold.

To avoid this problem, &vpol. periodically blocks

out &vvd. peek notification.

This is done when the peek rate is very high

and when no busy activity has been reported

for a time.

.cp 5

USER PROPERTIES HELP TEXT

The following section explains the idle detection

advanced properties at a level appropriate for

help text.

"Idle seconds allowed"

See "Idle sensitivity" for more details on idle detection.

When programs appear to be doing nothing but waiting for input,

the operating system gives them less time to run.  This is done

to give preference to programs that are doing useful work.

Some programs periodically appear to be waiting for input,

but then change their behavior and continue after a time.

A game may pause, for instance, to wait for the user to make

a choice, but then continue if the user does not react.

This property turns "Idle detection" off for a period of time

after useful work has been detected.  If a program appears

to run slowly when there is an option for the user to provide

input, try increasing this value.

This value is in seconds.  The default is no idle time allowed.

The property can be changed while the program is running

to tune it to the proper value.

"Idle sensitivity"

Also see "Idle seconds allowed."

DOS programs often "poll" for input when they are waiting

for a user response.  For instance, a program may wait for

a response by repeatedly checking to see if the user has hit

a key.	This repeated checking wastes time when other programs

could be running instead.  The operating system detects "idle" programs

by looking for a high rate of "polling" for input.  When programs

are judged to be waiting for input, they are given less time to run.

The idle sensitivity level is a threshhold for judging when

applications will be considered idle.  The value is the % of

the maximum possible polling rate the application could perform.

If an application polls at a rate higher than this value, it

is considered idle.

For example, if idle sensitivity is set to 75%, then an application

repeatedly checking to see if input is available would have to

do this checking at more than 75% of the maximum possible rate

before it would be judged idle.

Idle detection is a "best guess" of what the program is doing.

It could be that the program is polling at a very high rate,

but is still doing useful work as well.

The idle sensitivity threshhold

allows adjustment of the threshhold for a particular application.

If your application can receive input while running and

seems to run slower than expected, try adjusting the idle

sensitivity to a higher value.	This lets the application

poll at a higher rate without being judged idle.  Setting

the level to 100, turns idle detection off altogether.

If your application is just waiting for input and other

applications do not appear to be running, try adjusting

the idle sensitivity downward.	This lowers the threshhold

for judging the application idle.

The property can be changed while the program is running

to tune it to the proper value.  For most programs, the

default level is just right.

.*:h5.Data structure overview

.*:h4.Data Structures Description

.gml off

.fo off

.fo on

.gml on

.*:h4.Exported Interfaces

.gml off

.fo off

.fo on

.gml on

.*:h4.Imported Interfaces

.*:h4.Design Constraints

.*:h4.Design Review

.*:h3.&vpol. Implementation

.*:h4.Internal Interfaces

.gml off

.fo off

.fo on

.gml on

.*:h4.Implementation

.*:h4.Implementation Review

.*:h3.&vpol. Appendix

.*:h4.Glossary

.*:h4.Size and Performance Considerations

.*:h4.Implementation Estimates