.*      SCCSID = @(#)lpt.doc	6.2 91/04/11

.*

.***    lpt.doc - VLPT main document

.*

.*

.im ..\..\equate.sci

.im ..\..\macros.sci

.fo left

:h3.&vlpt. Architecture

.pi /Virtual LPT Device Driver

.pi /Virtual LPT/Architecture

:h4.Problem Description/Objectives

The &vlpt. component consists of virtual support for the

parallel line-printer output ports and for the printer-related

&cb. entry points.  It provides support in each &vdm. for up to

three parallel printers.

This component allows extant &dosapps. which access the parallel

ports, either by direct access to the ports, or by normal &cb.

calls, to operate without change.  This component also allows DOS

applications which perform screen-print operations using &cb.

INT 05h to operate without change.

:p.The &vlpt. also supports file transfer programs which utilize the

parallel ports, typically via special cables, to connect two systems together

in order to copy files from one machine to the other.  "Laplink" and

"Data Migration Facility" are two examples of this type of program.

To support this mode of operation, an interface is architected between

the &vlpt. and the Physical Printer Device Driver.

:h4.Solutions/Justification

:h5.Overview

From the perspective of the &vdm., the &vlpt. component supports

access via:

:ul

:li.&cb.

:li.I/O address space

:eul

:p.The Virtual Line PrinTer (VLPT) device driver supports &dosapp.

access to three virtual parallel port controller devices in the &cr.

&vdm. sessions.

:p.The &vlpt.  virtualizes the parallel port controller's Data port,

Status port, Parallel Control port, and Reserved port.

:p.The &vlpt. directly supports &cb. INT 17h, rather than allowing

the ROM (&cb.) to access the ports directly.

:histref refid=LPTHS01.

:p.The &vlpt. registers a direct interface with the Physical

Printer Device Driver (PLPT) to support Direct I/O Access.  To support

&dosapp.s which print via &cb. INT 17h services, VLPT

buffers the print data which is subsequently

directed to the file system services.

:p.To support DOS :hp1.file transfer programs:ehp1. on the parallel ports,

the &vlpt. also provides a :hp1.direct IO access:ehp1. mode for all &dosapp.s which

attempt to program the parallel port controller via direct I/O.

:histref refid=LPTHS03.

:h5.Print Screen Processing

:p.The &vlpt. also supports the Print Screen function.  &vlpt. hooks

INT 05h so that it gets notified before the &cb. INT

05h handler.  The &cb. INT O5h handler invokes INT 17h functions, and

the &vlpt. INT 17h emulation in turn sends this data to a

spool file, if the Spooler is active.  When the &cb. INT 05h handler

IRETs, the &vlpt. also

gets control (via VDHArmReturnHook) so that it may close the spool file,

if the Spooler is active.

:h5.Printer Close Processing

:p.The &vlpt. exports the VDHPrintClose service.  This interface may be

called by another &vdd. (ie.  Virtual Keyboard driver) to force any open

printers in a &vdm. to close (to handle Ctrl-Alt-PrtSc).  This service

is required because some DOS applications do not explicitly close or

disable the printer when their print activity is completed.

:p.The &vlpt. may also close open print files whenever

a &vdm. is terminated.  All open print files in the terminating &vdm.

are closed, after any buffered data has been sent to the Spooler.

:p.The &vlpt. will close the spool file after a timeout period which the user

can specify in an Advanced Property.   The default is used if the user

has not specified a value.   The user can specify a value of zero if

he doesn't want the time out feature to operate.   In which case

the &vlpt. will not close an open print file when the

&dosapp. which performed output to the printer terminates.  Depending

on the &dosapp.'s behavior, the &vdm. may remain active when the

program ends, and the Spooler print file may remain open.  If so, the

user can cause the open print file(s) to close using the

Ctrl-Alt-PrtSc control key sequence.

:h5.Interrupt-Driven &dosapp.s

:p.DOS applications that perform direct I/O to the parallel port

controller generally do not rely on hardware interrupt generation and

do not install hardware interrupt service routines.  LPT interrupts

(IRQ 7) are not reported in a &vdm..  In general, attempts by a

&dosapp.  to enable interrupts will be ignored, and the &dosapp.  will

never receive a hardware interrupt from the printer.

:histref refid=LPTHS02

:h5.File Transfer &dosapp.s

:p.There are two basic ways to transfer files on personal systems.  The

most common way has been using Asynchronous Communications (ie. RS232-C)

protocols and the COM device interface.  More recently, however, several

&dosapp.s have become very popular which perform file transfer using the

parallel port interface.  In order to support this latter class of file

transfer applications, the &vlpt. must support direct IO access operations.  Some

additional support is required for the IBM Data Migration Facility.

:h6.direct IO access Mode Operation

.pi /Virtual LPT/Direct I/O Access Mode

:p.All &dosapp.s which perform file transfer on the parallel ports do so by

programming the parallel port controller directly.  To support these

programs (and others which print via direct I/O),

:hp1.Direct I/O Access:ehp1.  mode is provided

by the &vlpt.  in order to give &dosapp.s direct access to the parallel

ports' Data, Status, and/or Control registers.

:p.These registers are not being fully virtualized by the &vlpt..  Rather,

when VLPT recognizes that a &dosapp.  wishes to directly access a parallel

port, it will request (via a direct VDD to PDD interface) that the Physical

Printer Device Driver (PLPT) grant VLPT temporary exclusive ownership of

the parallel port hardware.  If that port is not currently active (ie.

printing) under control of the PLPT, then PLPT will grant VLPT exclusive

access, and continue to service incoming File

System I/O requests.  Any incoming Monitor requests (ie. from the Spooler)

will be blocked until exclusive access is released (no error or status

monitor packets are sent to the monitor chain).

:p.If PLPT is actively processing a hardware I/O operation

(ie. servicing a monitor "Write" packet) when VLPT

makes a request for exclusive access, PLPT will give an appropriate

error code back to VLPT.  VLPT will then display a PopUp message

(via VDHPopUp) to allow the user to

select the most appropriate system action ("End program" or "Retry").

.*:histref refid=LPTHS12

:p.VLPT will continue handling traps on all direct IO access to the Control

and/or Reserved registers.  VLPT virtualizes the state of these registers

by selectively performing direct I/O operations to the hardware.  By

virtualizing the Control port, VLPT can recognize when a &dosapp.  is

enabling the Direction bit (bit 5).  Also, this provides the ability for

VLPT to "ignore" when a &dosapp.  attempts to set the "IRQ Enable" bit (bit

4).  Interrupts will not be simulated in the VDM.

:p.Since not all parallel port adapters support interrupts, very few

&dosapp.s are written to be interrupt-driven on parallel port I/O.  Rather,

&dosapp.s typically poll the parallel port controller registers to perform

file transfer, and in some cases, printing.  The &vlpt.  design is

sensitive to this, and does not include support for simulation of

interrupts to a &vdm..

:note.Due to the multitasking nature of &cr., and the necessity of running

file transfer programs in direct IO access mode, data communications using this type

of application will have increased probability of errors

when mutiple processes are concurrently active

and/or when the &vdm. is switched to the background.

.*:histref refid=LPTHS12.

:enote

:h6.Data Migration Facility

:p.The &vlpt. supports the IBM Data Migration Facility (DMF).  DMF will

run and access the parallel port hardware in the direct IO access mode of VLPT.

The following features of the Printer PDD (PLPT) and the &vlpt. (VLPT)

provide support for DMF:

:ul

:li.direct IO access Mode Operation

:p.As described above, direct IO access mode is provided by the &vlpt..  DMF

(Send and Receive) will run in direct IO access mode.

:li.&ps2. LPT Port Extended Mode Enabling

:p.On &ps2. systems, PLPT ensures that all LPT ports which support

Extended mode (ie. read/write 8-bit parallel I/O) will be enabled for Extended

Mode at system initialization time.  On any &ps2. models which do not enable

by default, PLPT will enable Extended Mode via the system Programmable

Option Select (POS) function.  This ensures that all &ps2. LPT ports will

support manipulation of the Control Port "Direction Control" bit

(for a description of this bit, see :hdref id=lptcpd.).

:eul

:note.The Receive program included with DMF (RECV35.COM) is programmed to

run only on IBM &ps2. systems, such as the &m80..  This restriction is

independent of the support provided for file transfer programs in VLPT.

:enote

:h5.&m80. Extensions

In the &m80. and compatibles, the planar parallel port has some extended

capabilities.

:ul

:li.Configurable I/O address space

:li.Bidirectional data flow

:eul

:h6.Configurable I/O Address Space

:p.A given &ps2.  parallel port can be configured to one of three

different address spaces:  03BC-Eh, 0378-Ah, or 0278-Ah.  This

configuration can be done via Programmable Option Select (POS).  The

&vlpt.  does not support &dosapp.s that attempt to configure the port

addresses via POS.  Such configuration changes are supported only by

booting the &m80.  Reference Diskette and selecting the Change

Configuration option.

:histref refid=LPTHS03

:h6.Bidirectional Capabilities

:p.The Extended Mode of the &ps2. parallel port allows the port to be

used as a bidirectional 8-bit parallel interface.  This capability is

supported by VLPT using the direct IO access mode.  The Printer PDD provides an

initialization function which ensures that all &ps2. systems have

the parallel port enabled for Extended Mode features.

:p.The IBM Data Migration Facility (DMF) is a &dosapp. which

allows users to transfer data from a &pcat. to a &ps2..  Support of

Extended mode features in a &vdm.  is provided only on &ps2. systems.

:histref refid=LPTHS09

:h5.Data Flow

.pi /Virtual LPT/Data Flow

The &vlpt. component data flow is shown in the following diagram:

:fig id=vlptorg place=inline frame=box

+---------+  +---------+   +---------+

| DOS App.|  | DOS App.|   |  CBIOS  |

+---------+  +---------+   | INT 05h |

| |PIO         |INT 17h   +------^--+

| |            |            |    |7

| |    +-------+  +---------+    +---------+

| |    |          |INT 17h                 |

| |    |          |                        |

| |    |          |           +---------+  |

| |    |          |           | Spooler |  |

| |    |          |           | Monitor |  |

| |    |          |           +-^-------+  | ring 3/2

| |    |          |             |  |       |

----|-|----|----------|-------------|--|-------|-----------

| |    |          |             |  |       |

| |    |   +------+             |  |       | ring 0

| |    |   |                    |  | 10    |

| |    |   |  +----------+   +-----V----+  |

| |    |   |  |   VDM    |-->|   OS/2   |  |

| |    |   |  | Manager  | 4 |  Kernel  |  |

| |    |   |  +-^--------+   +----------+  |

| |    |   |    |3            |    | 6     |

| |    |   |    |             |  +-V-----+ |

| |    |   |    |             |  |  LAN  | |

| |    |   |    |             |  | Spool | |

+--+ |    |   |    |             |  +-------+ |

|    |    |   |    |             |            |

|    |1   |2  |8   |             | 5          |

| +--V----V---V------+  11 +-----V-----+    +---------+

| |    LPT VDD       |<--->|  LPT PDD  |    | KBD VDD |

| +------------------+     +-----------+    +---------+

|                 |               |

+--------------+  |          +----+

|  | 12       | 13

+-V--V----------V------+

|  PARALLEL PORT HDWE  |

+----------------------+

:figcap.VLPT Component Organization

:efig

:h6.Scenario 1 -- Direct I/O Access to a Local Parallel Port Only

:ul

:li.The VDM is created.  VLPT handles the VDM_CREATE event.

:li.A &dosapp. performs any programmed I/O (PIO) to the parallel port

(ie. an OUT to the LPT1 "Data" port to output a character, or an OUT

to the LPT2 "Control" port to set the STROBE bit) [1].

This PIO instruction is trapped and routed to the VLPT and processed by its

OUT handler routine.

:li.Since this is the VDM's first PIO access to the port VLPT calls

the Physical Printer Device Driver (PLPT) with the "Request Direct Access"

command [11].  If the PLPT has no currently active output requests for

that parallel port, direct access is granted exclusively to the VLPT [12].

:li.If PLPT returns the "Exclusive Access Denied" error, indicating that

the port is currently active, then VLPT will display a PopUp message (via

VDHPopUp), giving the user the option to Retry,

Return error to the program, or to Terminate the VDM.

:li.If no error occurs on "Request Direct Access", VLPT disables I/O

trapping (by calling VDHSetIOHookState) on the Data and Status ports for

this LPT device, for this VDM only.  Attempts to perform direct I/O to this

LPT port in other VDMs will still be trapped, and will be handled by the

LPT I/O contention resolution logic of VLPT (see scenario 5).

:li.On all subsequent direct IO access to the parallel port's Data or

Status registers, each PIO instruction is executed directly

to the physical I/O register of the parallel port controller [9].

:li.On all subsequent direct IO access to the parallel port's Control or

Reserved registers, the hardware virtualization logic in the I/O trap

handler for the respective register is executed.  This is described in

more detail under :hdref id=lptdio..

:li.Since there is no buffering of Data or Status, and since the VDM's task

may be running concurrently with other tasks, there is some probability of

data errors.  This probability increases as the number of active processes

in the system increases, or when the VDM is made a background process.

:li.All Spooler Queue output to the parallel port which is being used to

support this VDM will be blocked by the Printer PDD until VLPT determines

that the &dosapp. running with direct IO access has been terminated.

:li.If the user attempts to invoke print screen functions via keystrokes

(Shift-PrtSc, Ctrl-PrtSc, or PrintScreen),

VLPT will process these requests (via INT

17h handling logic) even while concurrently supporting direct IO access mode

operations in the same VDM).

The spool job will remain in the appropriate Spooler Queue

until it can be output (when the parallel port is no longer dedicated

to VLPT direct IO access mode processing).

If the spooler is not active, the data will be interspersed with any

currently printing job.

:li.direct IO access operation continues in this manner until the &dosapp. stops

executing (ie. it's PDB is destroyed), or until the VDM is terminated.

At this time, VLPT calls PLPT with the "Release Direct Access" command.

This causes PLPT to resume normal operations with respect to File System

I/O requests and monitor activity.  The user retains responsibility for

changing the interface cabling, if necessary, to resume normal printer

output operation.  If the user fails to make necessary changes, the

appropriate error messages will be displayed.

:eul

:note.It may be necessary to adjust the priority of the VDM when it is

switched to the background while running in direct IO access mode.  This performance

tuning aspect of the VLPT implementation will be determined by the

development team responsible for VLPT.

.*:histref refid=LPTHS12

:enote.

:h6.Scenario 2 -- INT 17h Access to a Local or Remote Printer

:ul

:li.DOS application invokes INT 17h to access the printer.  This event

generates a trap that is handled by the 8086 Emulation routines.

Since the LPT VDD hooks this &cb.  interrupt, its INT 17h handler is

invoked [2].

:li.The &vlpt. INT 17h handler emulates the &cb. routines.  On

the first INT 17h print character request, the LPT VDD will

call the VDM Manager to process an open request (VDHOpenPRN) [3].

If an error is returned from VDHOpenPRN, the &vlpt.  will display a PopUp

(via VDHPopUp), giving the user the option to Retry, Return error to

the program, or to Terminate the VDM.

:li.&vdmm. issues a file system OPEN request [4]

which returns a handle.

:li.The VLPT initializes control structures and an output buffer

for this LPT device within this VDM.  Printer initialization is always

handled during open processing.

:li.On subsequent INT 17h "Print Character" requests, the LPT VDD

will queue characters in its output buffer until the buffer is filled.

When this buffer fills, the LPT VDD calls the VDM Manager's VDHWritePRN

function [3].  VDHWritePRN is always called at VDM task time.

:li.VDM Manager routes print stream data to the File System [4], where

it is written to the LPT PDD [5] (in the case of a Local printer), or

to the LAN Spooler [6] (in the case of a network printer).

If an error occurs on VDHWritePRN (eg.

WRITE_FAULT), the &vlpt. will display a PopUp with the options:

:ul

:li.retry

:li.return error to program

:li.terminate the VDM

:eul.

:p.From the perspective of VLPT, there is no distinction between a

DOS application and any other VDM code that executes INT 17h

functions.  For instance, another &cb. routine may issue calls to

the INT 17h interface [8].  Also, any software interrupt handler which

is chained into the INT 17h service chain will be executed in a

manner fully compatible with DOS.

:li.Printing continues in this manner until the printer is closed

(refer to the section below on Printer Close Detection).  Then the

LPT VDD issues a call to VDHClosePRN [3] to release the handle to the

printer.

:eul

:h6.Scenario 3 -- INT 05h (Print Screen) to Local or Remote Printer

:ul.

:li.From within any VDM session, when the user invokes the keyboard

combination Shift-PrtSc, the Virtual Keyboard device driver (VKBD)

detects this sequence and reflects an interrupt (INT 05h) into that

VDM [7].

:li.The &vlpt. sets a hook for INT 05h, and its handler is invoked at

the time of the call (before the &cb. routine) [8].  At this time the

LPT VDD hooks the INT 05h IRET (VDHArmReturnHook) to receive control

when the &cb. handler completes.

:li.The &cb.  INT 05h handler (and/or any other installed INT 05h

handler) gets invoked and in turn calls the INT 17h interface [8].

:li.The LPT VDD INT 17h emulation then behaves as described under

Scenario 2, above.

:li.Upon IRET from the &cb. INT 05h handler, the &vlpt. completes

the print screen processing by closing the opened file.

(VDHClosePRN) [3].

:eul

:h6.Scenario 4 -- IBM Data Migration Facility

:ul.

:li.During system initialization on &ps2.s, the Printer PDD (PLPT) will enable

the parallel ports' Extended Mode features via Programmable Option Select.

:li.At VDM Creation time, VLPT hooks the four parallel controller I/O ports

for each of the LPT devices present.

:li.During VDM run time, VLPT will detect the first direct IO access to a port and

proceed to operate in direct IO access mode, as described in Scenario 1 above.

:eul

:note.DMF "Receive" (RECV35.COM) support is provided only on &ps2. systems

because non-&ps2. systems do not have the required parallel port hardware.

On non-&ps2. systems, only the DMF "Send" function is operational.  This is

due to the way RECV35.COM is programmed and not a limitation of the &vlpt..

On non-&ps2. systems, the initialization processing described above is

likewise not performed by PLPT.

:histref refid=LPTHS09

:enote.

:h6.Scenario 5 -- Multi-VDM I/O Contention Resolution

:ul

:li.One VDM is performing I/O to a parallel port on behalf of a &dosapp.

using either the &cb. INT 17h emulation or the direct IO access mode support

features described above.

:li.A second VDM attempts to perform I/O to the same parallel port.

:li.If the port in question is already active in direct IO access mode, then:

:ul

:li.If the second VDM attempts to emulate &cb. functions, and the Spooler

is active, then the normal &cb. emulation logic will resolve the contention

and generate a Spool file.  VLPT will be able to support both operations

concurrently.

If the spooler is not active, VLPT will display a PopUp message (via VDHPopUp)

requiring user intervention.

:li.If the second VDM attempts to use the port in direct IO access mode, then

VLPT will display a PopUp message (via VDHPopUp), suspending the

second VDM which is attempting direct IO access.  The PopUp will have the following

optional user responses:

:ul

:li.Return error to program (Default)

:li.Retry

:li.Terminate the VDM

:eul.

:eul.

:li.If the port in question is already active emulating &cb., then:

:ul

:li.If the second VDM is attempting a &cb. operation, then VLPT will

provide the standard &cb. emulation.  This will result in a second

Spool job, to which VLPT will send data via (VDHWritePRN).  VLPT can

handle both VDMs concurrently.

If the spooler is not active, the data from the two VDMs will be interspersed

on the printer.

:li.If the second VDM attempts to use the port in direct IO access mode,

VLPT will attempt to enable direct IO access for the second VDM.  If that port

is not already in use (ie. printing) then direct IO access mode will be enabled,

and both VDMs will succeed in their respective operations.

:eul.

:eul.

:h5.Status Indication

:p.The &vlpt.  must also provide various status indicators to the

&vdm..  These indicators can be simulated entirely within the &vlpt..

This support is provided for the &cb. INT 17h function 02h (Read

Status) emulation.

:p.The &cb. STATUS flags being emulated are described in the IBM Publication

:hp2.Personal System/2 and Personal Computer BIOS Interface Technical

Reference:ehp2. under "Interrupt 17H - Printer".

The &vlpt. emulates the INT 17h Read Status return value

appropriately based on return from VDHOpenPRN or VDHWritePRN interfaces.

:p.The "NOT BUSY" bit indicates, in general, that the &dosapp. can

print another character.

:h5.Interrupt Service

:p.The &vlpt. component does not support virtual hardware

interrupts.  If a &dosapp. attempts to enable interrupts (ie.

attempts to set CONTROL port bit 4, "IRQ EN"), that I/O will be

ignored, and the application will not receive interrupts from the

parallel port hardware.

:histref refid=LPTHS05

:h4.Architectural Review

:h3.&vlpt. Design

.pi /Virtual LPT/Design

:h4.Design Overview

:p.The execution control of the &vlpt. is divided into seven

logical modules:

:ol

:li.&vlpt. Initialization

:li.&vdm. Creation

.br

:li.direct IO access Mode

:li.&cb. INT 17h Emulation

:li.Print Screen

:li.Printer Close Detection

:li.&vdm. Termination

:eol

:h5.&vlpt. Initialization

:p.During system initialization, after the physical printer driver has

been installed and initialized, the VDM Manager calls the &vlpt.

initialization routine.  The &vlpt.  then determines, by examining the

BIOS 40&gml. data area, the number and location of parallel ports

installed.

The &vlpt. registers event handler routines for VDM Creation and VDM

Termination events (via VDHInstallUserHook).   The exported

entry point, VDHPrintClose, is a dynamic link entry point which

does not have to be registered.

:figref refid=LPTHD01. for a detailed list of the I/O port addresses.

:p.The &vlpt. registers its VDD-PDD interface entry point (via

VDHOpenPDD).  At this point the dynamic link references between the

Physical Printer Device Driver and the &vlpt. are fully resolved.  This

interface is utilized to support direct IO access mode operations.

:h5.&vdm. Creation

:p.Upon creation of the &vdm., the &vlpt.

configures the relevant portions of the &cb.  data area

and initializes the internal states of the virtual parallel

ports.  For this &vdm., I/O handler routines are registered

(VDHInstallIOHook), and &cb. INT 17h emulation routines are installed

(VDHInstallIntHook).  An INT 05h handler is also registered for

supporting PrintScreen function.

:h5 ID=lptdio.Direct I/O Access Mode

.pi /Virtual LPT/Direct I/O Access Mode

:p.Each parallel port controller provides these four I/O ports:

:ul

:li.8-bit Data Port (Direct Access - Input/Output)

:li.Control Port    (Virtualized - Input/Output)

:li.Status Port     (Direct Access - Input Only)

:li.Reserved Port   (Virtualized - I/O Ignored)

:eul

:p.For a more detailed description of the layout of these ports, see

:hdref id=LPTHD01..  Also,

refer to the IBM Publication :hp1.Personal System/2 Hardware Interface

Technical Reference:ehp1.

:h6.Data and Status Port Direct Access

:p.If the first I/O access to a parallel port in a VDM is to the Data

or Status Port, that I/O will be trapped.  Once direct IO access mode operations

are fully initialized, I/O operations to Data and Status ports will

no longer cause traps.  Therefore, subsequent I/O to these ports will not

result in VLPT getting control.  VLPT does get control, however, on

any I/O to the Control or Reserved ports, as described below.

.*

:p.In direct IO access mode the 8-bit Data port is directly accessed by the VDM.

On a &pcat. system the Data port always operates in Compatible mode.

On a &ps2., the Physical Printer driver initializes the parallel ports

to operate in Extended hardware mode.

For a detailed description of Data Port behavior, see :hdref id=lptdpd..

.*

:p.The Status port indicates the status conditions of the parallel

port hardware.  In direct IO access mode, a read performed on this port will reveal

the state of the parallel controller's Status port at that moment.  Writes

to the Status port have no effect.

For a detailed description of Status port behavior, see :hdref id=lptspd..

:h6.Control and Reserved Port Virtualization

:p.When any code executing in the context of a VDM session performs

direct I/O to either the Control or Reserved ports, a trap will be

generated and the VLPT handler for that particular event is invoked.

There are two separate handlers (for IN instructions and OUT instructions)

for each I/O port being hooked.

.*

:p.The Control port is virtualized in order to maintain control over

and awareness of the state of each bit in the port.

:ul

:li.BIT 7,6 - Reserved

:li.BIT 5 - Direction Control

:li.BIT 4 - Interrupt Enable

:li.BIT 3 - Select In (Pin 17) Control

:li.BIT 2 - Initialize Printer (Pin 16) Control

:li.BIT 1 - Auto Feed XT (Pin 14) Control

:li.BIT 0 - Strobe (Pin 1) Control

:edl.

For a more detailed description of the virtualized behavior

of the Control Port bits, see :hdref id=lptcpd..

.*

:p.VLPT hooks all traps generated by I/O to the Reserved port.  Reads from

this port will always return the value FFh.  Writes to

this port are ignored and have no effect on the behavior of the other

parallel controller I/O ports.

:histref refid=LPTHS10.

:h6.VLPT to PLPT Direct Interface

:p.Following the architecture for the direct VDD-PDD interfaces, VLPT

obtains the PDD interface entry point using VDHOpenPDD service at

VDD initialization time.  This interface incorporates the following

functions (which are also implemented by the PLPT):

:ul

:li.PDDCMD_REGISTER  --  To register the VLPT entry point with PLPT

:li.PDDCMD_DEREGISTER  --  To notify PLPT at MVDM Shutdown

:li.PLPT_REQUEST_EXCLUSIVE  --  To request exclusive access

:li.PLPT_RELEASE_EXCLUSIVE  --  To give up exclusive access

:eul

.*:p.For a more detailed description of the VDD-PDD interface for the

.*&vlpt., please see :hdref id=lptddi..

:h5.&cb. INT 17h Emulation

.pi /Virtual LPT/INT 17h Emulation

:p.The &vlpt. component also supports &cb.  access through

software interrupt 17h (23).  Rather than allowing the &cb.  to

perform the functions, this component performs the functions

directly.  This support is similar to the virtual I/O port access

support, although simpler.

:p.On the first instance of an INT 17h from any &vdm., or when there

is not already an open Spool file for this port in this VDM, then

the &vlpt. will issue an implicit open request (via VDHOpenPRN).

:p.The &cb.  access emulation supports

three functions:

:ul

:li.Print Character

:lp.The Print Character function simply puts the character in the

LPT VDD output buffer. The virtual status of the port is returned.

:li.Initialize

:lp.The Initialize function calls the &vdd. with a special

initialize command.  The virtual status of the port is returned,

and any open print file on that device is closed.

:li.Read Status

:lp.The Read Status function simply returns the status of the

virtual parallel port.  This status, as well as the status

returned by the Print Character and Initialize functions are

slightly different from the status read from the virtual parallel

status port.

:eul

If the Spooler is not active, the VDHOpen/Write/ClosePRN routines

will pass through via VDHSetError, any errors that the Physical

device driver can return.

The following equates and return codes are the possible error codes which

can be returned by device drivers (\drv6\inc\error.inc).

.fo off

ERROR_I24_WRITE_PROTECT         EQU     0

ERROR_I24_BAD_UNIT              EQU     1

ERROR_I24_NOT_READY             EQU     2

ERROR_I24_BAD_COMMAND           EQU     3

ERROR_I24_CRC                   EQU     4

ERROR_I24_BAD_LENGTH            EQU     5

ERROR_I24_SEEK                  EQU     6

ERROR_I24_NOT_DOS_DISK          EQU     7

ERROR_I24_SECTOR_NOT_FOUND      EQU     8

ERROR_I24_OUT_OF_PAPER          EQU     9

ERROR_I24_WRITE_FAULT           EQU     10

ERROR_I24_READ_FAULT            EQU     11

ERROR_I24_GEN_FAILURE           EQU     12

ERROR_I24_DISK_CHANGE           EQU     13

ERROR_I24_WRONG_DISK            EQU     15

ERROR_I24_UNCERTAIN_MEDIA       EQU     16

ERROR_I24_CHAR_CALL_INTERRUPTED EQU     17

ERROR_I24_NO_MONITOR_SUPPORT    EQU     18

ERROR_I24_INVALID_PARAMETER     EQU     19

ERROR_I24_DEVICE_IN_USE         EQU     20

.fo on

The printer device driver currently may return the following error codes

(\drv6\src\dev\printer\prtdd.inc).  In the future, the kernel printer device

driver may change to use any of the architected set of device driver return

codes as documented above.

.fo off

ERROR_I24_BAD_UNIT              EQU     1

ERROR_I24_NOT_READY             EQU     2

ERROR_I24_BAD_COMMAND           EQU     3

ERROR_I24_OUT_OF_PAPER          EQU     9

ERROR_I24_WRITE_FAULT           EQU     10

ERROR_I24_GEN_FAILURE           EQU     12

ERROR_I24_CHAR_CALL_INTERRUPTED EQU     17

ERROR_I24_INVALID_PARAMETER     EQU     19

.fo on

The following diagram show what error codes may be returned based upon

the DOS or VDH call issued and whether a character monitor (spooler) is

registered.  This information is based on the CRUISER PS/2 printer device

driver.  The CRUISER AT printer device driver should pretty much hold

true to this table also.

.fo off

+-------------+-----------------------+------------------------+

| VDH CALL    |   SPOOLER ENABLED     |    SPOOLER DISABLED    |

+-------------+-----------------------+------------------------+

| VDHOpenPRN  | NO ERRORS RETURNED    |  NO ERRORS RETURNED    |

+-------------+-----------------------+------------------------+

| VDHWritePRN | CHAR_CALL_INTERRUPTED |  CHAR_CALL_INTERRUPTED |

|             | WRITE_FAULT           |  BAD_UNIT              |

|             |                       |  OUT_OF_PAPER          |

|             |                       |  WRITE_FAULT           |

+-------------+-----------------------+------------------------+

| VDHClosePRN | CHAR_CALL_INTERRUPTED |  NO ERRORS RETURNED    |

|             | WRITE_FAULT           |                        |

+-------------+-----------------------+------------------------+

.fo on

:h5.Print Screen

.pi /Virtual LPT/Print Screen

:p.At VDM creation, the LPT VDD registers an INT 05h handler with

8086 Emulation.  This handler is invoked prior to the &cb. INT 05h

handler.  It opens the PRN device (LPT1) for that VDM and causes the

screen printout to be sent to a spooler file.

If the spooler is not active, the data will be sent to the printer and

not to a spooler file.

:p.The IRET from INT 05h is also hooked, using VDHArmReturnHook.  This

allows the &vlpt. to determine when to close the spooler file opened

at the beginning of the PrintScreen activity.

:p.Since PrintScreen activity is INT 17h activity, it can be supported

even from a VDM which is active in direct IO access mode supporting a file transfer.

The screen dump will be spooled via the INT 17h logic of the &vlpt.

if the spooler is active. Otherwise, it will go directly to the printer.

:h5.Printer Close Detection

:p.In a conventional environment, when a &dosapp. writes to the

printer, the character is immediately sent to the physical

printer.  In the &vdm. environment, for INT 17h support, data is first

buffered temporarily in the &vdd., then sent to the file system

via VDHWritePRN service.

If the spooler is active, the data is always written first to the

disk spool file, and the Spooler sends the data to the LPT PDD

which sends the data to the device.

If the spooler is not active, the data is sent directly to the printer.

:p.Spooling programs typically need to know when a printer is

opened and closed, so that printer data may be broken up into

files and printed on a file-by-file basis.  Unfortunately, &cb.

does not normally provide an indication when a printer is closed.

This problem is also shared by &dos. spoolers and networked

printers.  When the printer is accessed through the &cb.  or

through direct I/O, rather than through INT 21h, three methods are

provided to close the printer

:histref refid=LPTHS07.&gml.

:ol.

:li.Automatically Via a Time Out

:lp.Each time the application performs an INT 17h a timer is disarmed

and then rearmed.   Should the &dosapp. discontinue writing the the

printer via int 17 the timer will timeout and the spool file will be

closed.    The user can specify the timeout value to be used via the

Advanced Properties user interface either before the application is

started or whilst it is running.   The user can set a timeout value

of Zero in which case it will never use the automatic timeout and the

user will have to use one of the following two methods.   The system

will ship with a default time out.   A separate timer is used for each

LPT so that an app which is printing to multiple LPTs at the same time

will have the spool files closed on inactivity to that LPT.

:li.User Command -- Ctrl+Alt+PrtSc

:lp.This keyboard sequence allows the user to close the printer on

command.  The Keyboard &vdd. calls the VDHPrintClose service exported

by the &vlpt..  This service then closes any open print data streams

in that &vdm..  Subsequent INT 17h or direct I/O to the parallel control

port (STROBE bit) will automatically cause the &vlpt. to re-open the printer

(VDHOpen).

:li.VDM Termination

:lp.The &vlpt. will set an event hook (VDHInstallUserHook) with

VDM Manager to be notified on the termination of a VDM.  If

there are any open print streams in this VDM, the &vlpt. will close

each of them.

:eol

:p.Closing a print data stream will cause any data remaining in the

&vlpt. buffer for that stream to be printed (via VDHWrite).   The spool file

close timer will be stopped.

Then the open device handle will be closed (via VDHClose).

:h5.&vdm. Termination

:p.As just described above, all data remaining in the &vlpt. print

buffers when the VDM is terminated

will be written out before the spool file is closed. If the spooler is

not being used, the remaining data will be sent to the printer.

All active I/O routines for the &vdm.

are unhooked (VDHRemoveIOHook).  All other per-VDM resources are

deallocated.

:h4.Data Structures Description

:h5 ID=LPTHD01.Parallel Controller I/O Ports

.pi /Virtual LPT/I/O Ports

:p.Each parallel port controller provides four I/O ports:

:ul

:li.8-bit Data Port (Direct Access - Input/Output)

:li.Control Port    (Virtualized - Input/Output)

:li.Status Port     (Direct Access - Input Only)

:li.Reserved Port   (Virtualized - I/O Ignored)

:eul

:p.All supported parallel controllers for &pcat. and &ps2.

systems provide these four I/O ports.  Although the &vlpt. will

allow &dosapp.s to read the Data port, only &ps2. systems support

Extended Mode, which allows remote systems to present 8 bits of data

which can be read by performing input on the Data port.  This &ps2.

parallel port hardware feature is fully supported by the &vlpt.

:p.If a direct I/O instruction to any of these four ports is the

first I/O on that parallel controller from a given &vdm., then

the I/O trap handler will call PLPT with the "Request Exclusive" command.

:p.These four I/O ports are in four adjacent byte addresses, where the

base address is determined by the controller number (LPT1, LPT2, or

LPT3).  The base addresses supported are the same for the &pcat.  and

the &ps2..

:histref refid=LPTHS10.

:p.For example, on a &pcat. configured with one

Monochrome/Printer adapter and two Serial/Parallel adapters, the I/O

port addresses supported by the &vlpt. are shown in

:figref refid=LPTFG01 page=no..

:fig id=LPTFG01 frame=none place=inline

.ti > 09

.tb   17 26 36

.bx 16 25 35 49

>      >Port   >I/O

>Device>Name   >Address

.bx

>LPT1>Data    >3BCh

>    >Status  >3BDh

>    >Control >3BEh

>    >Reserved>3BFh

.bx

>LPT2>Data    >378h

>    >Status  >379h

>    >Control >37Ah

>    >Reserved>37Bh

.bx

>LPT3>Data    >278h

>    >Status  >279h

>    >Control >27Ah

>    >Reserved>27Bh

.bx off

:figcap.Parallel Controller I/O Port Addresses

:efig

.ti 3E 3E

:p.The I/O port addresses for a &ps2. with three parallel ports

installed will also appear as shown in

:figref refid=LPTFG01 page=no..

:h6 refid=lptdpd.8-Bit Data Port (Input/Output)

:p.The 8-bit data port is used to indicate the data byte that is to be

transferred to/from the parallel device.  This byte is always accessed

directly by the VDM, rather than being virtualized.  This eliminates

the overhead associated with I/O trap handling.  On a &pcat. system,

the Data port always operates in Compatible mode.  On a &ps2. system,

the Data port operates in Extended mode.  The following describes this

distinction in detail.

:ul

:li.In Compatible mode, Writes to the port cause 8 bits of data to be

immediately presented to the port's connector pins.  Read operations

in compatible mode always present the data previously written to the port.

.*

:li.In Extended mode, the behavior depends on how the Control port's

"Direction Control" bit is set:

:ul

:li.If the Direction Control bit is ON (enabled), a write to the Data port

will only latch the data, but will not present this data at the connector

pins.  A read from the Data port will return the data on the connector pins

sent from a remote device.

:li.If the Direction Control bit is OFF (disabled), a write to the Data port

will latch the data and present this data at the connector pins.  A read

from the Data port will return the data last written to it.

:eul

:eul

:h6 refid=lptspd.Status Port     (Input Only)

:p.The Status Port is always a read-only port on both &pcat. and &ps2.

systems.  The bits in the status

port are numbered from 0 to 7.  Bit 0 is the least significant

bit.  Values written to this port are always ignored by the hardware.

:hp1.Access to this port is always processed in direct IO access mode.  This

means that the values of each bit are represented to the &dosapp.

directly from the parallel controller, without any virtualization

processing by the &vlpt.:ehp1.

:ul

:li.BIT 0, 1

:lp.These bits are reserved.  The value of these bits is always 1.

:li.BIT 2 (-IRQ STATUS)

:lp.This bit indicates if an interrupt is pending.  Since enabling

LPT interrupts is not supported, this bit is always 1.  See the

Control Port definition, :hdref refid=lptcpd..

:li.BIT 3 (-ERROR)

:lp.This bit indicates the state of the printer -ERROR signal.

The initial value of this bit is 1.

:li.BIT 4 (SELECT)

:lp.This bit indicates the state of the printer SELECT

signal.  The initial value of this bit is 1.

:li.BIT 5 (PE)

:lp.This bit indicates the state of the printer PE (paper

end) signal.  The initial value of this bit is 0.

:li.BIT 6 (-ACK)

:lp.This bit indicates the state of the printer -ACKNOWLEDGE signal.

The initial value of this bit is 1.

:li.BIT 7 (-BUSY)

:lp.This bit indicates the state of the printer -BUSY signal.

The initial value of this bit is 1.

:eul

:h6 refid=lptcpd.Control Port    (Input/Output)

:p.Like the data port, the control port may be read or written.

However, unlike the Data or Status ports, the &vlpt. always traps on

direct IO access to the Control port.  This allows certain events to be virtualized,

as described in the list below.

:p.When read, the Control port presents the value that was last written to it.

:p.When written, the behavior of each bit is determined by the state of the

hardware (the &vlpt. in most cases executes the identical instruction

directly to the hardware port).  For certain bits, as indicated below, the

&vlpt. disallows the operation, and the particular bit modification is

ignored.  The bits in the Control Port are numbered from 0 to 7.

Bit 0 is the least significant bit:

:ul

:li.BIT 0 (STROBE)

:lp.All reads and writes to this bit are passed unchanged to and from

the hardware.

:li.BIT 1 (AUTO FD XT)

:lp.All reads and writes to this bit are passed unchanged to and from

the hardware.

:li.BIT 2 (-INIT)

:lp.All reads and writes to this bit are passed unchanged to and from

the hardware.

:li.BIT 3 (SLCT IN)

:lp.All reads and writes to this bit are passed unchanged to and from

the hardware.

:li.BIT 4 (IRQ EN)

:lp.Since interrupt-driven print or file transfer &dosapp.s are not

supported in a &vdm., the &vlpt. always MASKS OFF the value of this bit

before performing an OUT to the hardware.  The value of this bit is

always 0.

:li.BIT 5 (Direction)

:lp.Since Extended mode is supported on &ps2. systems, &dosapp.s may

modify this bit in direct IO access mode.  The hardware responds as follows:

:ul

:li.When set, reading the Data port provides a data byte supplied by a remote

connected device.

:li.When reset, reading the Data port provides the 8 bits written

to it by the immediately preceeding write.

:eul

:li.BIT 6, 7 (Reserved)

:lp.When read, these bits are always ON (set).  Values written to these

bits are ignored (MASKED OFF) by the &vlpt.

:eul

:h6.Reserved Port   (I/O not supported)

:p.The fourth I/O port on each parallel controller is reserved,

and is always hooked by the &vlpt. in order to virtualize the port.

:p.If this port is read from, the value FFh will always be returned.

If this port is written to, that output will be ignored.  Direct

I/O to this port is not supported, and will have no effect

on operations to the other parallel controller I/O ports.

:histref refid=LPTHS10.

:h5.&cb. Data Area

:p.The system global &cb. Data Area is referenced by the

&vlpt. during its initialization.

:figref REFID=LPTFG03 page=no.  lists each &cb. data item

maintained by the &vlpt. component on a per-VDM basis.  The offsets

shown are relative to the &cb. data segment (0040:0000):

:fig id=LPTFG03 frame=none place=inline

.ti > 09

.tb  2 11 23

.bx 1 10 22 69

>      >Initial

>      >Value

>Offset>(Hex)    >Description

.bx

.in 23

.ir 5

.un 21

.*            |23456789x123456789x123456789x123456789x1|

>0008 >Varies  >LPT1 Base.  Normally initialized to

.un 21

>         >the base I/O address of the first

.un 21

>         >printer port found. (WORD)

.bx

.un 21

>000A >Varies  >LPT2 Base. (WORD)

.bx

.un 21

>000C >Varies  >LPT3 Base. (WORD)

.bx

.un 21

>000E >0000    >Reserved. (WORD)

.bx

.un 21

>0010 >Varies  >Installed Hardware.  Bits 14 and 15

.un 21

>         >reflect the number of Printer Adapters

.un 21

>         >installed. (WORD)

.bx

.un 21

>0078 >Varies  >LPT1 Timeout Counter.  This value

.un 21

>         >reflects the amount of time the CBIOS

.un 21

>         >will wait if the device is busy. (BYTE)

.bx

.un 21

>0079 >Varies  >LPT2 Timeout Counter. (BYTE)

.bx

.un 21

>007A >Varies  >LPT3 Timeout Counter. (BYTE)

.bx

.un 21

>007B >0000    >Reserved. (BYTE)

.bx off

.in

.ir

:figcap.&vlpt. Component &cb. Data Items

.ti 3E 3E

:efig

:p.If the I/O address for an LPT port is zero (0), then the

corresponding port is not available.

The &vlpt. will

initialize each &vdm.'s &cb. data area so that all three parallel

port base addresses are filled in.

The value for the Reserved areas (40:000Eh, 40:007B) is always zero.

:p.On both the &pcat. and &ps2. systems, the port base address can

vary due to hardware configuration (the number and type of

parallel port adapters installed).  On the &ps2., the base address can

be configured using the &ps2. Reference Diskette setup program.

:p.Because all three parallel ports are virtualized the

Installed Hardware flags, bits 14-15, will be equal to three.

:p.The Timeout Counter value is set for each installed parallel port,

and corresponds to the period of time the &cb. routine waits before

reporting "Busy" status.  This value varies depending upon the speed

of the CPU, and may be changed on a per-VDM, per-printer basis by

&dosapp.s.

:h4 refid=lptei.Exported Interfaces

.pi /Virtual LPT/Exported Interfaces

:vdhlist

.im lptvdh.vdh

:evdhlist

:h4.Imported Interfaces

.im lptimp.sci

:h4.Design Constraints

:ol

:li.Spooler does not need to be active

:lp.The print spooler need not be active to print from a Virtual DOS

Machine using CBIOS functions. If the Spooler is not active, there may

be data from multiple print jobs being intermixed.

:lp.Printing from &dosapp.s in direct IO access mode is supported

even where the Spooler is not configured for output to a parallel port.

:lp.Printing via the File System (Standard I/O) is available

in a &vdm., regardless of whether spooling is active or not.

:li.Interrupts Are Not Supported.

:lp.Due to the deficiency of many extant controllers, this is

considered an acceptable deficiency.  Very few &dosapp.s attempt

to hook printer interrupts.

:li.Timing Is Different

:lp.No attempt has been made to insure that real-time behavior

matches that of the real DOS environment, beyond gross performance

objectives.

:lp.Timing errors may occur in applications which perform direct I/O

to parallel devices if the &vdm. is switched to the background or when

the system load (ie. number of processes, interrupt latency, etc)

is high.

.*:histref refid=LPTHS12.

:li.AT Serial/Parallel Adapter Shadowing Registers Not Supported

:lp.The shadowing behavior of the upper four I/O addresses on the

AT Serial/Parallel adapter (0x7C-0x7F, where x = 2 or 3) is not

virtualized by the &vlpt.

:lp.It is not believed that any applications are dependent on the

shadowing behavior of the AT Serial/Parallel Adapter.

:histref refid=LPTHS10.

:eol

:h4.&vlpt. Advanced Properties

:h5.LPT Close Timeout (secs)

:p.When a user prints from his DOS application in the DOS operating

system, the user must only tell the application to print and data starts

printing on the printer.  When a user prints from his DOS application

in the DOS session of OS/2, the user tells the application to print

but if the application prints using interrupt 17h, then the user must

either terminate the application, terminate the VDM, or press

Ctrl-Alt-PrtSc to flush the OS/2 interrupt 17h print buffer,

to tell the spooler to close the spool file and to make the spool

file ready to print.

:p.Most users are unaware of this function and do not understand

they must do something different from their application when running

under OS/2 than when they are running under DOS.

:p.It is not possible with &dosapps. that print via INT 17 to tell

when they have completed their output so that we can close the spool

file.

:p.The basic solution is to have the virtual printer device driver set

a timer when a DOS application is printing via interrupt 17h.  Each

interrupt 17h print request will cause the timer to be reset.  If

interrupt 17h print requests stop, the timer will expire and the virtual printer

device driver will flush its buffer and close the printer handle,

transparent to the user.

:p.An Advanced property is there to allow the user to

select a print timeout value in seconds.   A setting of Zero will

mean No timeout (for applications which produce output very slowly).

The default will be set to 15 seconds with a maximum value of 1 Hour.

The property can be set on a per application basis and whilst the application

is running.   The same timeout value will be used for LPT1, LPT2, LPT3

in that VDM but each VDM can have a different timeout value.   Each

LPTx (1-3) will have a different timer so that each will timeout independently

based on the time they were last written to.

The dialog box looks like the following:

.fo off

+--------------------------------------------+

|             DOS Properties                 |

+--------------------------------------------+

|                                            |

|  Property: +--------------------------+---+|

|            | LPT close timeout (secs) | V ||

|            +--------------------------+---+|

|                                            |

|                                            |

|  Value:    +-------+ --------------------- |

|            |    15 | |<       | |       >| |

|            +-------+ --------------------- |

|                                            |

|                                            |

|                                            |

|  < OK  >   < Default>   <Cancel>  < Help > |

|                                            |

+--------------------------------------------+

.fo on

:p.The user should be able to select a device, set the timer on or off

and enter the timeout value between 0 and 3600.  The property will be

saved in the usual way when the user hits the OK button.

If no property has been set a default of 15 seconds will be used.

:h4.Design Review

Held 2/21/89 and 9/7/89.

:h3.&vlpt. Implementation

:note.See the &cr. &vlpt. I0 and I1 documents for this level of detail.

:enote.

.*:h4.Internal Interfaces

.*:h4.Implementation

.*:h4.Implementation Review

:h3.&vlpt. Appendix

.*:h4.Glossary

.*:h4.Size and Performance Considerations

.*:h4.Implementation Estimates

:h4.Design History

.pi /Virtual LPT/Design History

.im lpthis.his