.*	SCCSID = @(#)mouse.doc	  11.12 90/10/08

.*

.* symbol definitions

.im ..\..\equate.sci

.im ..\..\macros.sci

.*

:Bissues.mouseisu.isu

.*

.*

.*

.*

:h3.&vmd. Architecture

.*

.*

:h4.Problem Description/Objectives

No known applications talk directly to mouse hardware, given the

diversity of mouse hardware and the complexity of dealing with all

the combinations of mouse and video equipment.	Since we then assume that

all applications that support a mouse do so through the Int 33h interface,

the problem is limited to providing full Int 33h functionality to &mvdms..

The Int 33h interface performs the following&gml.

:dl compact tsize=5

:dt..

:dd.Position and button tracking

:dt..

:dd.Position and button event notification

:dt..

:dd.Selectable pixel and mickey mappings

:dt..

:dd.Video mode tracking

:dt..

:dd.Video pointer management (location and appearance)

:dt..

:dd.Light pen emulation

:dt..

:dd.Miscellaneous features

:dt..

:dd.Driver state saveability, restoreability, and disability

:edl

There should be no restrictions on any use of the Int 33h interface, even

when a VDM is &bgd..  The only difference between &bgd., focused and &fgd.

VDM support should be that no mouse events are posted for a &bgd. VDM.

Mouse events are posted to full screen VDM from the physical mouse device

driver (PMD) and are posted through the PM shield to focused windowed VDMs.

Pointer position should remain unchanged as long as a VDM is &bgd., unless

an application initiates pointer movement of its own.  In that case, virtual

pointer position will change, but nothing will be drawn (not even in virtual

video space).

:histref refid=novirt

.*

.*

:h4.Solutions/Justification

.*

:h5.Int 33h support

Int 33h support is provided directly by the Virtual Mouse driver (VMD), using

a standard VDM software interrupt hook.  This support is fundamentally the

same as what was previously provided in the real-mode portions of the Physical

Mouse drivers (PMDs), but is converted to run in protected-mode, in order to

save space and share code in all VDMs.	This support is driven in three basic

ways&gml. from interrupt-time PMD notifications for full screen VDMs, from

task-time VDM requests for Int 33h services, and from task-time mouse event

notification of the shield for the focused windowed VDMs.  The VMD maintains

all the per-VDM mouse state information itself.

:histref refid=supp33

The interface between PMD and VMD is straightforward.  The PMD is aware at

all times of which session owns the mouse;  thus, when a &fgd. full screen

VDM owns the mouse, it notifies the VMD of mouse events.  The events themselves are

buffered by the VMD until they can be simulated to the VDM at which time

the Int 33h data structures for the &fgd. VDM is updated.

If a window VDM is in focus, the PMD will route the mouse events to the

PM single queue driver then to the shield and eventually to the VMD.  The

VMD then buffers the mouse event the same way as if it was for the full

screen VDM. When the advanced property 'Exclusive mouse access' is ON

for a window VDM, all mouse events will go directly from PMD to VMD,

bypassing all the PM. This property is necessary for those applications

that track and draw their own mouse pointer.

.*

:h5.Position and button data tracking

Position and button events are interrupt-time events that are communicated

to the VMD via a "post mouse event" entry point.  The mouse event processing

is delayed until task time.  For performance reasons, all adjacent motion

events in the event buffer are coalesced.

Normally, the VDM will not be processing any events, so position and button

information can be immediately retrieved and stored for later query by a VDM

application, via Int 33h.  This is because most apps derive their mouse

support by polling Int 33h.

.*

:h5.Position and button event notification

Each time the VMD is notified of a mouse event, it may have to notify a VDM

user-defined mouse event subroutine.  This depends on whether or not a VDM

application has registered such a subroutine (via an Int 33h function), and

what events the caller has asked to be notified of.

When a registered subroutine must be called, it is done by requesting a

fake interrupt to be simulated into the VDM.  A fake interrupt service

routine (loaded into each VDM at create-time) immediately EOIs the interrupt

and then proceeds to notify the VDM's registered subroutine.  In order to

pick an IRQ that is guaranteed to not conflict with any other VDD, the VMD

queries the PMD at init-time for the physical IRQ.  It is not important to

have the real mouse IRQ, simply an IRQ that no other VDD will need.

:histref refid=intsim

For movement-only events, we coalesce them.  As events are posted from the

PMD, the VMD compares it with the previous event in the event buffer (if any),

and if it is a movement-only event, the current event cumulates the previous

event in the buffer.  This is continued until the next event is not a

movement-only event or the movement event has been simulated.  This reduces

the amount of interrupt-simulation overhead during rapid event generation

periods.

.*

:h5.Selectable pixel-to-coordinate and mickey-to-pixel mappings

Pixel-to-coordinate and mickey-to-pixel mappings are maintained entirely

by VMD.  PMD only reports raw mickey mouse events to VMD for full screen

VDMs.  For windowed VDMs, PM shield reports absolute pixel mouse events

to the VMD.

The VMD takes raw delta mickeys reported and maps them to the VDM's

current virtual-coordinate grid.

.*

:h5.Video mode tracking

The VMD must be notified of changes in the physical dimensions of the screen,

and use that information to map the raw mickeys into pixel coordinates.

The VVD notifies the VMD of mode changes with a device-independent interface,

so that the VMD is capable of supporting virtually any video device without

change.

:histref refid=modechg

.*

:h5.Video pointer management

Interfaces are provided to the VMD by the VVD to handle the following

operations&gml.

:dl compact tsize=5

:dt..

:dd.Define pointer.  This describes the desired pointer shape, and

is always issued as a prelude to making a pointer visible for the

first time.  An error may be returned if the VVD is unable to

provide pointer support for the current video mode, allowing the VMD to

skip further drawing/moving requests.  This is not strictly necessary

however;  the VVD will continue to refuse calls it cannot support.

:dt..

:dd.Draw pointer.  A position given in pixel coordinates is provided.

For text modes, the VVD maps chunks of pixels to individual character

locations.  The size of a "chunk" is 8x8, unless the mode is 40-column, in

which case it is 16x8.	Furthermore, a video-page parameter is provided,

to support the Int 33h Set CRT Page service.  Draw requests without any

intervening remove requests are treated as "move" requests.

:dt..

:dd.Remove pointer.

Exactly what the name implies.

:edl

:histref refid=drawptr

.*

:h5.Light pen emulation

Whenever this Int 33h feature is enabled, a VMD Int 10h hook is

established, and Int 10h Light Pen functions emulated with mouse position

and button data.

.*

:h5.Miscellaneous features

The Int 33h interface provides a function to read-and-reset "mouse motion

counters", which are simply software counters of total horizontal and vertical

mickeys.  These are simulated within the VMD, by taking reported pixel

movements and converting them to mickeys (based on the current mickey-to-pixel

ratio).

The interface must also provide counts of button presses and releases, and

the positions at the time of the last press and release.  These too are

simply maintained by the VMD, without any work on the PMD's part.

.*

:h5.Driver state save, restore, and disable

These are trivial to support.  Services are provided to copy all per-VDM

mouse state information to an application-defined block and back again.  When

the driver is disabled, it ignores all Int 33h requests and PMD events.

.*

:p.

For a complete description of all the supporting Int 33h interfaces,

refer to the Microsoft Mouse Programmer's Reference

(available from MS Press, ISBN 1-55615-191-8).

.*

.*

:h4.Architectural Review

To be performed.

.*

.*

.*

.*

:h3.&vmd. Design

.*

.*

:h4.Design Overview

.*

.*

:h4.Data Structures Description

To be added.

.*

.*

:h4.Exported Interfaces

.*

:h5.Exported data declarations

.im1 mseext.sin

.*

:h5.Exported procedure interface flag definitions

.im1 mseexk.sin

.*

:h5.Exported procedure headers

.im1 mseexp.sin

.*

.*

:h4.Imported Interfaces

.im1 mseimp.sin

.*

.*

:h4.Design Constraints

To be added.

.*

.*

:h4.Design Review

To be performed.

.*

.*

.*

:h3.&vmd. Implementation

.*

.*

:h4.Internal Interfaces

.*

:h5.Local data declarations

.im1 mselot.sin

.*

:h5.Local procedure interface flag definitions

.im1 mselok.sin

.*

:h5.Local procedure headers

.im1 mselop.sin

.*

.*

:h4.Implementation

To be added.

.*

.*

:h4.Implementation Review

To be performed.

.*

.*

.*

:h3.&vmd. Appendix

.*

.*

:h4.Glossary

To be added.

.*

.*

:h4.Size and Performance Considerations

To be added.

.*

.*

:h4.Implementation Estimates

To be added.

.*

.*

:h4.Design History

.im mousehis.sci

.*

:Eissues.mouseisu.isu